Module 1

Fundamentals of Operational Technology (OT)

Student Lab Workbook

Industrial Cybersecurity Training Program

| Trainee name |  |
| --- | --- |
| Team / Kit ID |  |
| Cohort / Class |  |
| Trainer |  |
| Start date |  |

# Contents

Contents	1

How to Use This Workbook	2

Working Method for Every Session	3

Evidence Naming Convention	3

Safety-First Rules (apply to the whole module)	3

Kit Parameter Sheet	4

Module 1 Learning Outcomes	5

Session 1: Understanding the OT Environment and Asset Landscape	6

Scenario	7

Learning Objectives	7

What you will use this session	7

Pre-Flight Checklist	7

Exercise Steps	7

Part A - Asset Identification (trainer-led rack tour + kit walk-through)	8

Part B - Purdue and Zone Mapping	8

Suggested AI prompt	9

Part C - Traffic Capture and Communication Matrix	10

Part D - Asset Criticality Ranking	11

Suggested AI prompt	11

Deliverables - tick when complete and evidence is saved	12

Reflection Questions	13

Evidence Log - Session 1	13

Session 2: OT Traffic Capture and Protocol Deep-Dive	14

Scenario	15

Learning Objectives	15

What you will use this session	15

Pre-Flight Checklist	15

Exercise Steps	15

Part A - SPAN / Monitor Port Configuration	16

Part B - Capture and Protocol Survey	16

Part C - Packet Deep-Dive on the Selected Protocol	17

Part D - Protocol Security Assessment	18

Suggested AI prompt	19

Deliverables - tick when complete and evidence is saved	20

Reflection Questions	20

Evidence Log - Session 2	21

Session 3: Interpreting OT Incidents Through Operational Evidence	21

Scenario	22

Learning Objectives	22

What you will use this session	22

Pre-Flight Checklist	22

Exercise Steps	22

Part A - Case Study Analysis	22

Part B - Device-to-Case Study Mapping	23

Part C - AI-Assisted Attack-Path Reconstruction	23

Suggested AI prompt	24

Part D - Indicators, Escalation Triggers and Defensive Controls	25

Deliverables - tick when complete and evidence is saved	26

Reflection Questions	26

Evidence Log - Session 3	27

Session 4: Performing Threat Using STRIDE	27

Scenario	28

Learning objectives	28

What you will use this session	28

Pre-flight checklist	28

Exercise steps	28

Part A: Scope and decomposition	29

Part B: STRIDE threat identification	29

Part C: Existing controls and gaps	31

Part D: Mitigation plan	31

Part E: Challenge the threat model with AI	32

Deliverables: tick when complete and evidence is saved	33

Reflection questions	34

Evidence Log: Session 4	34

Module 1: Completion Record	35

Sign-Off	36

# How to Use This Workbook

Module 1 establishes the foundations of assessing an operational technology (OT) environment, progressing over four sessions from asset identification and traffic analysis to incident interpretation and structured threat modelling. This workbook is the structured record for that work: each session sets out the scenario, objectives, procedure and templates, and the completed tables, diagrams, captures and integrity hashes together constitute the assessed evidence bundle for the module. Work is carried out by the team against the Trainee Kit, supported by Trainer Rack demonstrations and shared services on the Central infrastructure.

## Working Method for Every Session

- Read the scenario and objectives for the session before touching any tool.

- Complete the pre-flight checklist. Do not start the exercise until every item is ticked.

- Work through the numbered steps in order. Where a command, filter or prompt is given in a grey box, use it as written unless the trainer changes the placeholder value.

- Fill in the tables as you go, not from memory afterwards. Distinguish observed facts from assumptions.

- Capture evidence using the naming convention below and log it in the evidence table.

- Use Generative AI only where a step directs you to. Treat its output as material for your own analysis, never as a finding in itself: validate every claim against your own evidence — packet captures, observed device behavior, the asset inventory and architecture, the incident case pack, and the OT protocol reference guide. Record where the AI was wrong, unsupported or over-generalised.

- Get the trainer sign-off at the end of the session.

## Evidence Naming Convention

M1-W<session>-<KIT_ID>-<step>-<short-description>.<ext> Examples: M1-W1-KIT03-05-purdue-mapping.png M1-W2-KIT03-04-span-capture.pcapng M1-W4-KIT03-09-stride-register.xlsx

Record every artefact in the Evidence Log at the end of each session. For any exported file, compute a SHA-256 hash and record it.

# Windows PowerShell: hash an evidence file Get-FileHash -Algorithm SHA256 '.\M1-W2-KIT03-04-span-capture.pcapng'

## Safety-First Rules (apply to the whole module)

- No changes without backup, rollback plan and trainer approval. This includes switch/firewall configuration, host settings and any active tooling.

- The SIS (Safety Instrumented System) island is out of scope for any scan, write, injection or configuration change at all times. Observe it only through passive evidence.

- Passive first. In Module 1 the default posture is observation: capture, read, document. Active discovery (Nmap) in Session-1 style tasks is host/inventory-oriented and must stay within the trainer-approved target list.

- Stop and call the trainer if any device alarms, drops communication, changes state unexpectedly, or if you are unsure whether an action is safe.

- Generative AI is a challenger, not a source of truth. Never paste credentials, live IP schemes you were told to keep internal, or client-identifying data into an external AI tool.

# Kit Parameter Sheet

Complete this table before Session 1 and refer back to it throughout the module.

Your trainer will give you the values for your kit and the Trainer Rack. Use these placeholder names wherever the steps refer to them.

| Placeholder | Value (fill in) | Description | Notes |
| --- | --- | --- | --- |
| <KIT_ID> |  | Your kit / team identifier | Used in evidence file names |
| <PLC_IP> |  | PCS 7 CPU 410 controller IP | Control zone |
| <ET200SP_IP> |  | ET 200SP I/O station IP | Control zone |
| <HMI_WS_IP> |  | Operator / engineering workstation IP |  |
| <SC646_MGMT_IP> |  | SCALANCE SC646-2C firewall mgmt IP | Zone boundary device |
| <XC208_MGMT_IP> |  | SCALANCE XC208G switch mgmt IP | Access switch |
| <KIT_SUBNET> |  | Kit subnet / mask (e.g. 192.168.10.0/24) |  |
| <KIT_GW> |  | Kit default gateway |  |
| <OT_VLAN> |  | OT / control VLAN ID |  |
| <SUPPORT_VLAN> |  | Maintenance / engineering VLAN ID |  |
| <SPAN_SRC_PORTS> |  | Switch ports mirrored to the monitor port | Session 2 |
| <SPAN_DST_PORT> |  | Monitor (SPAN) port for Wireshark host | Session 2 |
| <TR_PCAP_NAME> |  | Shared Trainer Rack PCAP file name | Distributed each session |
| <TR_RTU_IP> |  | SICAM A8000 RTU IP (Trainer Rack) | Reference only |
| <TR_RELAY_IP> |  | SIPROTEC 5 relay IP (Trainer Rack) | Reference only |
| <TR_SIS_SEG> |  | Trainer Rack SIS island segment / VLAN | OUT OF SCOPE for active work |
| <AI_TOOL> |  | Approved Generative AI tool / URL | Validation assistant only |
| <EVIDENCE_STORE> |  | Path / share for the evidence bundle |  |

# Module 1 Learning Outcomes

By the end of Module 1 you will be able to:

- Identify and classify OT assets on a Trainee Kit and Trainer Rack, and place each on the Purdue model with an operating zone/VLAN.

- Capture OT network traffic safely using a SPAN/monitor port and analyse it in Wireshark at protocol and packet level.

- Build a communication-flow matrix and protocol-usage matrix from real captured evidence.

- Explain the security characteristics and weaknesses of common OT protocols and propose compensating controls.

- Reconstruct real OT incidents (Stuxnet, Ukraine, TRITON, Colonial Pipeline) and map their stages to lab assets and defensive controls.

- Perform a structured threat and risk assessment of an OT architecture segment using STRIDE, and produce a ranked risk register.

- Use Generative AI as a challenger and validate its output against operational evidence, identifying hallucinations.

# Session 1: Understanding the OT Environment and Asset Landscape

Total duration: 120 minutes

## Scenario

The trainer introduces one undocumented asset (an asset card or a prepared capture containing an unidentified device). Your team must identify, classify, and place it correctly using the evidence you generate during the session. Everything you conclude must be backed by something you observed.

## Learning Objectives

- Identify each asset on your Trainee Kit and the demonstrated Trainer Rack assets.

- Map every asset to a Purdue level and an operating zone/VLAN.

- Build a communication-flow matrix from a shared PCAP.

- Produce a validated asset-criticality ranking.

- Use AI to challenge your Purdue placements and criticality, and prove at least one AI claim wrong.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Trainer Rack | Rack tour, shared asset reference, OT traffic capture source |
| Trainee Kit | Hands-on asset identification and Purdue/zone mapping |
| Wireshark / PCAP | Traffic capture review; IP/MAC/protocol analysis; communication matrix |
| Interactive 3D model | Visual understanding of process and asset relationships |
| Generative AI (<AI_TOOL>) | Challenge Purdue placement and asset-criticality assumptions |

## Pre-Flight Checklist

☐ Kit powered up; all devices show healthy status LEDs.

☐ Kit Parameter Sheet completed for your kit.

☐ Wireshark opens on the workstation and can see at least one interface.

☐ Shared Trainer Rack PCAP <TR_PCAP_NAME> received and opens without error.

☐ Asset cards for Trainer Rack assets received.

☐ Access to <AI_TOOL> confirmed.

☐ Evidence folder created at <EVIDENCE_STORE> using the naming convention.

## Exercise Steps

### Part A - Asset Identification (trainer-led rack tour + kit walk-through)

- Follow the trainer's rack tour. On the Trainer Rack, locate and note the function of: CPU 410 PLC, ET 200SP HA I/O, SIL3 Safety I/O, SICAM A8000 RTU, SIPROTEC 5 relay, SCALANCE switches, and review the interactive 3D model.

- Walk through your own Trainee Kit and physically identify: PCS 7 CPU 410 controller, ET 200SP I/O modules, SCALANCE SC646-2C firewall, SCALANCE XC208G switch, and the workstation.

- Record every asset in the Asset Inventory below. Add the Trainer Rack assets from the shared asset cards. Note the SIS island-mode isolation from the BPCS.

Table W1.1 - Asset Inventory

| # | Asset name / model | Location (Kit / Rack) | Role / function | IP address | MAC address | Firmware / version | Source of info |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |

### Part B - Purdue and Zone Mapping

- Map each asset to its Purdue level (0-5). Include the Trainer Rack SIS as an island-mode Level 1 segment with dedicated segmentation.

- Assign each asset to an operating Zone. Record Purdue level, zone, connected interfaces and network segment.

- Use <AI_TOOL> as a devil's advocate: ask it to challenge two of your Purdue/zone placements. Then defend or correct each mapping using operational evidence (what the device does, what it talks to).

⚠ SAFETY Do not accept an AI reclassification without evidence. If AI says 'the RTU belongs at Level 2', check what it actually communicates with in the PCAP before changing anything.

#### Suggested AI prompt

Act as a devil’s-advocate OT architecture reviewer. I will provide an asset, its function, its communication relationships, and my proposed Purdue-level and zone placement. Challenge my proposed placement rather than validating it by default. For the asset below: - Identify the strongest argument against the proposed Purdue-level and/or zone placement. - Explain why that argument matters from an OT architecture perspective. - Identify the specific evidence or documentation that would confirm or refute the argument. - Distinguish clearly between facts provided, reasonable interpretation, and assumptions. Do not assume facts that I have not provided. If the available information is insufficient to determine the correct placement, explicitly state what information is missing rather than inferring it

Table W1.2 - Purdue / Zone Mapping

| Asset | Purdue level | Zone | Connected interfaces | Network segment | Placement rationale (evidence) |
| --- | --- | --- | --- | --- | --- |

Table W1.3 - AI Challenge Log: Purdue / Zone

| Placement challenged | AI's counter-argument | Your evidence | Outcome (kept / corrected) |
| --- | --- | --- | --- |

### Part C - Traffic Capture and Communication Matrix

- The trainer captures ~15 minutes of live OT traffic on the Trainer Rack with Wireshark and distributes one shared PCAP (<TR_PCAP_NAME>) to all kits. Open it in Wireshark.

- Survey the capture. Use Statistics ▸ Conversations and Statistics ▸ Protocol Hierarchy to see who talks to whom and with what.

- Analyse the PCAP for IP addresses, MAC addresses, ports, protocols and packet frequency. Useful starting points are given in the box below.

Statistics > Endpoints (all IPv4 / Ethernet endpoints) Statistics > Conversations (sort by packets / bytes; note duration) Statistics > Protocol Hierarchy (% of traffic per protocol) Statistics > I/O Graph (periodicity of cyclic OT traffic) Display filter examples: ip.addr == <PLC_IP> (Traffic to/from the specified PLC IP address) s7comm ¦¦ cotp (S7 / PCS 7 controller traffic) modbus (Modbus/TCP) arp (who-has / address discovery) eth.addr == <suspect MAC> (Traffic to/from the specified MAC address)

- Build the Communication Flow Matrix: every device-to-device flow, its protocol, port(s), approximate frequency/period, and its operational purpose.

Table W1.4 - Communication Flow Matrix

| Source (IP / role) | Destination (IP / role) | Protocol | Port(s) | Direction | Frequency / period | Purpose |
| --- | --- | --- | --- | --- | --- | --- |

- Identify the undocumented asset introduced by the trainer. Record its IP, MAC, the protocols it uses, what it talks to, and your best classification. State what evidence would confirm the classification.

Table W1.5 - Unidentified Asset Finding

| Attribute | Observation |
| --- | --- |
| IP address |  |
| MAC address |  |
| OUI vendor |  |
| Protocols seen |  |
| Peers (who it talks to) |  |
| Likely device type |  |
| Likely Purdue level & zone |  |
| Confidence & confirming evidence |  |

### Part D - Asset Criticality Ranking

- Ask <AI_TOOL> to rank the assets by criticality to the process. Identify at least one hallucinated dependency in its answer, prove it wrong using the PCAP, and correct the ranking.

#### Suggested AI prompt

Given the asset list and communication matrix, rank the assets by operational criticality based on the potential impact of losing each asset on the process. For each asset: - Assign an operational criticality ranking from highest to lowest. - Provide a one-sentence justification for the ranking. - State the specific process or system dependency the ranking relies on. - Clearly distinguish between dependencies directly supported by the provided communication matrix and those that are inferred. Do not assume a dependency that is not supported by the provided information. Where the available evidence is insufficient to determine an asset’s operational criticality, explicitly state the uncertainty and identify what additional evidence would be required. I will verify each stated dependency against packet-level communication evidence.

- Record the validated criticality ranking. Flag which Trainer Rack assets would be most disruptive to lose and why.

Table W1.6 - Asset Criticality Ranking (validated)

| Rank | Asset | Consequence if lost | Key dependencies (evidence-checked) | AI error corrected? (Y/N + note) |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

## Deliverables - tick when complete and evidence is saved

☐ Asset Inventory (≥ all kit assets + demonstrated rack assets)

☐ Purdue / Zone Mapping with rationale column complete

☐ AI Challenge Log: at least two placements challenged

☐ Communication Flow Matrix built from <TR_PCAP_NAME>

☐ Unidentified asset finding with confidence statement

☐ Validated Asset Criticality Ranking with at least one AI hallucination documented

☐ Evidence Log updated; screenshots named and hashed

## Reflection Questions

Which asset was hardest to place on the Purdue model, and what evidence finally settled it?

What in the capture told you the SIS island is isolated from the BPCS?

Where did the AI's reasoning break down, and what would have made its answer trustworthy?

## Evidence Log - Session 1

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 1 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 2: OT Traffic Capture and Protocol Deep-Dive

Total duration: 120 minutes

## Scenario

The trainer provides baseline traffic together with one controlled protocol anomaly, for example an unexpected write, a diagnostic request, a new communicating node, a replayed sequence, or an unusual message frequency. The trainer selects the protocol to focus on; you are not expected to cover every protocol.

## Learning Objectives

- Configure and verify a SPAN / monitor port on the SCALANCE XC208G.

- Capture live OT traffic safely and identify protocols by OSI layer and Purdue level.

- Dissect packets of the selected protocol: operation type, addressing model, and transaction identifiers.

- Assess protocol security properties and known weaknesses.

- Produce an OT Traffic Documentation Table and a Protocol Security Summary with compensating controls.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| SCALANCE XC208G | SPAN configuration and monitoring-port setup |
| Wireshark | Live capture; protocol hierarchy; conversations; display filters; packet inspection |
| OT Protocol Reference Guide | Protocol comparison by OSI layer, Purdue level, ports, security features |
| Trainer Rack | Shared traffic source and switch configuration demonstration |
| Generative AI (<AI_TOOL>) | Explain protocol usage; support protocol-security interpretation |

## Pre-Flight Checklist

☐ SPAN pre-check done with trainer; you know which ports are <SPAN_SRC_PORTS> and <SPAN_DST_PORT>.

☐ Current switch configuration exported as a rollback baseline (trainer-supervised).

☐ Monitoring host cabled to <SPAN_DST_PORT>; NIC up; Wireshark ready.

☐ OT Protocol Reference Guide to hand.

☐ Trainer has told you which protocol is in scope this session.

☐ Backup / rollback / approval confirmed before any switch change.

## Exercise Steps

### Part A - SPAN / Monitor Port Configuration

- Watch the trainer open the SCALANCE XC208G interface on the Trainer Rack and walk through SPAN configuration.

- On your kit switch, with trainer approval, configure the monitor session: mirror <SPAN_SRC_PORTS> to <SPAN_DST_PORT>. Record source ports, monitoring port and traffic direction (RX / TX / both).

⚠ SAFETY Mirroring adds load to the destination port only. Do not mirror onto a port that carries live control traffic and never mirror the SIS segment.

- Connect the workstation running Wireshark to <SPAN_DST_PORT> and verify packet flow; you should see traffic you are not the source or destination of.

Table W2.1 - SPAN Configuration Record

| Field | Value |
| --- | --- |
| Switch (model / mgmt IP) |  |
| Source port(s) mirrored |  |
| Monitor / destination port |  |
| Direction (RX / TX / both) |  |
| Config change approved by |  |
| Rollback baseline file |  |
| Packet flow verified (Y/N, how) |  |

### Part B - Capture and Protocol Survey

- Capture live OT traffic for 15 - 20 minutes and save the PCAP (M1-W2-<KIT_ID>-04-span-capture.pcapng).

- Use Statistics ▸ Protocol Hierarchy to identify the OT/ICS protocols present and their share of traffic volume.

- Complete the Protocol Usage Matrix. For every protocol observed: OSI layer, Purdue level(s) where it is used, port(s), whether it is encrypted/authenticated, and which lab devices use it.

Table W2.2 - Protocol Usage Matrix

| Protocol | OSI layer | Purdue level(s) | Port(s) | Encryption / auth? | Lab devices using it | % of capture |
| --- | --- | --- | --- | --- | --- | --- |

### Part C - Packet Deep-Dive on the Selected Protocol

- Apply a Wireshark display filter for the selected protocol and isolate protocol-specific traffic. Examples and anomaly-hunting filters are given below.

modbus modbus.func_code == 6 (write single register) s7comm s7comm.param.func (S7 job / function) cotp ¦¦ tpkt cotp.type (ISO-on-TCP transport for S7) dnp3 dnp3.al.func (DNP3 application function) iec60870-104 iec60870_asdu.typeid (IEC 104 ASDU type identifier) enip ¦¦ cip cip.service (CIP service code) Anomaly hunting: frame.time_delta_displayed > 2 (gaps in cyclic traffic) tcp.flags.syn == 1 && tcp.flags.ack == 0 (new connections / new node) modbus.func_code >= 5 (control writes vs reads 1-4)

- Examine packet headers, payload structure, the message or service type, addressing and identifiers. The exact terms depend on the protocol; use the mapping below to find the equivalent for the protocol in scope.

- Complete the Packet Dissection Worksheet for 3 to 5 representative packets, covering a spread of message types: at least one data read / poll, one write, command or control, and one diagnostic, management or session-setup message (or the closest equivalent your protocol has).

Table W2.3 - Packet Dissection Worksheet

| Frame # | Src → Dst | Message / service type (protocol term) | Category (read / write-control / diagnostic / setup) | Target address / object / point | Value / payload summary |
| --- | --- | --- | --- | --- | --- |

- Document the function types observed across the capture: reads, writes, diagnostics, configuration requests. Note anything that looks like the trainer's injected anomaly.

Table W2.4 - Observed Function Types & Anomaly Note

| Function type | Count / frequency | Normal or unexpected? | Evidence (frame #s) |
| --- | --- | --- | --- |

### Part D - Protocol Security Assessment

- Identify the selected protocol's security characteristics: authentication, encryption, integrity protection, replay protection, and known weaknesses.

- Compare two observed protocols across addressing model, function capabilities, security features and typical deployment scenario.

Table W2.5 - Protocol Comparison

| Attribute | Protocol A: ____________ | Protocol B: ____________ |
| --- | --- | --- |
| Addressing model |  |  |
| Read functions |  |  |
| Write / control functions |  |  |
| Authentication |  |  |
| Encryption |  |  |
| Integrity |  |  |
| Known weaknesses |  |  |
| Typical Purdue level |  |  |

- Use <AI_TOOL> to explain why the specific protocols are used for their specific lab purposes. Validate every claim against the OT Protocol Reference Guide and your capture.

#### Suggested AI prompt

Explain why <protocol> is used for <observed purpose> between <device A> and <device B> at Purdue Level <X>. Cover the following aspects: - Determinism – How the protocol supports predictable or time-sensitive communication. - Overhead – Its bandwidth, processing, and communication overhead. - Addressing – How devices and data are identified or addressed. - Security Trade-off – Key security strengths, limitations, and inherent trade-offs associated with using the protocol in this context.

- Populate the OT Traffic Documentation Table: IPs, ports, device roles, protocols, function types and operational purpose for every significant flow.

Table W2.6 - OT Traffic Documentation Table

| Src IP | Dst IP | Src role | Dst role | Protocol | Port(s) | Function types | Operational purpose |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Create the Protocol Security Summary: for each protocol, its vulnerabilities and the compensating controls that would reduce the risk in this architecture.

Table W2.7 - Protocol Security Summary

| Protocol | Vulnerability / weakness | Realistic attack in this lab | Compensating control(s) |
| --- | --- | --- | --- |

- Restore the switch configuration to the exported baseline (trainer-supervised). Confirm the SPAN session is removed and normal operation is verified.

## Deliverables - tick when complete and evidence is saved

☐ SPAN Configuration Record complete, with rollback baseline referenced

☐ Saved PCAP from your own SPAN capture

☐ Protocol Usage Matrix

☐ Packet Dissection Worksheet (3 - 5 packets, mixed function types)

☐ Observed function types & anomaly note: anomaly identified

☐ Protocol Comparison (two protocols)

☐ OT Traffic Documentation Table

☐ Protocol Security Summary with compensating controls

☐ Switch configuration restored and verified; Evidence Log updated

## Reflection Questions

What was the trainer's injected anomaly, and which single filter or statistic exposed it fastest?

For the selected protocol, which one control would give the biggest security improvement for the least operational risk?

What could a SPAN port miss entirely, and how would you cover that gap?

## Evidence Log - Session 2

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 2 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 3: Interpreting OT Incidents Through Operational Evidence

Total duration: 120 minutes

## Scenario

You receive a short OT incident brief and selected operational evidence. Your team must reconstruct the likely access path, the affected assets, the observable indicators, and the defensive controls, without reproducing malware or safety-system behavior.

## Learning Objectives

- Analyse four landmark OT incidents and extract sector, access vector, exploitation, lateral movement and OT impact.

- Map real-world incident targets to your lab assets.

- Use AI to reconstruct attack paths and challenge those reconstructions with evidence.

- Identify observable indicators of compromise (IoCs) for OT.

- Build a Defensive Control Mapping Matrix and define escalation triggers, especially for SIS anomalies.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Incident Case Study Pack | Stuxnet, Ukraine Power Grid, Triton/TRISIS, Colonial Pipeline |
| Trainer Rack inventory | Map lab assets to real-world incident targets and defensive controls |
| Device-to-Case Mapping Template | Document sector, attack path, impacted assets, observable indicators |
| Generative AI (<AI_TOOL>) | Reconstruct incident paths and challenge lab-to-incident mappings |

## Pre-Flight Checklist

☐ Incident Case Study Pack received (all four cases).

☐ Session 1 Asset Inventory and Trainer Rack inventory to hand.

☐ Device-to-Case Mapping Template available.

☐ Access to <AI_TOOL> confirmed; reminder: no malware, no exploit code, reconstruction only.

## Exercise Steps

### Part A - Case Study Analysis

- Review the four OT incident case studies: Stuxnet, Ukraine Power Grid (2015/2016), Triton/TRISIS, Colonial Pipeline.

- For each case, document: sector, access vector, exploitation, lateral movement, and OT impact.

Table W3.1 - Incident Case Analysis

| Incident | Sector | Initial access vector | Exploitation | Lateral movement | OT / process impact |
| --- | --- | --- | --- | --- | --- |
| Stuxnet |  |  |  |  |  |
| Ukraine Power Grid |  |  |  |  |  |
| Triton (TRISIS) |  |  |  |  |  |
| Colonial Pipeline |  |  |  |  |  |

### Part B - Device-to-Case Study Mapping

- Using the Trainer Rack inventory, identify which lab devices map to real-world incident targets (e.g. safety controller ↔ TRITON; RTU/relay ↔ Ukraine; engineering workstation ↔ Stuxnet; IT/billing boundary ↔ Colonial Pipeline).

- Complete the Device-to-Case Study Mapping.

Table W3.2 - Device-to-Case Study Mapping

| Lab device | Maps to incident | Real-world target it represents | Why this mapping holds | Exposure in our lab |
| --- | --- | --- | --- | --- |

### Part C - AI-Assisted Attack-Path Reconstruction

- Use <AI_TOOL> to reconstruct the Ukraine Power Grid attack path and map each stage to lab assets. Record the output, then challenge it against the case pack.

- Use <AI_TOOL> to reconstruct the Colonial Pipeline incident path and identify where SIS island mode would help contain or limit impact.

#### Suggested AI prompt

Reconstruct the publicly reported attack path for <incident> as an ordered sequence of stages, from initial access through impact. For each stage, provide: - Attacker Action – What the attacker did. - Target System Class – The type of system or asset targeted. - Observable Indicator – One indicator that a defender could potentially have observed. - Mapped Lab Asset – The corresponding asset from the provided lab asset list: <paste asset list>.

Table W3.3 - AI Attack-Path Reconstruction: Ukraine Power Grid

| Stage | Attacker action | Target system class | Mapped lab asset | Observable indicator | Verified vs case pack? (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |

Table W3.4 - AI Attack-Path Reconstruction: Colonial Pipeline

| Stage | Attacker action | Target system class | Mapped lab asset | Where SIS Island mode helps | Verified? (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |

### Part D - Indicators, Escalation Triggers and Defensive Controls

- Document the observable IoCs from the AI-generated attack paths: network, host and OT-process indicators. Note which of these your Module 1 tooling could actually detect.

Table W3.5 - Observable IoC Table

| IoC | Type (net / host / process) | Which incident | Detectable with our tools? How? |
| --- | --- | --- | --- |

- Discuss and record escalation triggers: the conditions that would move an event from 'monitor' to 'incident'. Give SIS anomalies their own row; any unexpected SIS state change or communication is an immediate escalation.

Table W3.6 - Escalation Triggers

| Trigger condition | Why it matters | Immediate action | Who is notified |
| --- | --- | --- | --- |
| SIS anomaly - any unexpected state change or communication |  |  |  |

- Create the Defensive Control Mapping Matrix: for each incident stage class, the preventive, detective and responsive controls, and whether that control exists in the lab today.

Table W3.7 - Defensive Control Mapping Matrix

| Attack stage class | Preventive control | Detective control | Responsive control | Present in lab? (Y / Partial / N) |
| --- | --- | --- | --- | --- |

## Deliverables - tick when complete and evidence is saved

☐ Incident Case Analysis: all four cases complete

☐ Device-to-Case Study Mapping

☐ AI Attack-Path Reconstruction: Ukraine Power Grid (with verification column)

☐ AI Attack-Path Reconstruction: Colonial Pipeline (with SIS-island analysis)

☐ Observable IoC Table

☐ Escalation Triggers, including a dedicated SIS row

☐ Defensive Control Mapping Matrix

☐ Note of at least one AI reconstruction error corrected against the case pack

## Reflection Questions

Which lab asset is your 'TRITON equivalent', and what single control most reduces that risk?

For the Ukraine reconstruction, which stage would your Session 2 monitoring have caught, and which would it have missed?

Where did the AI over-state certainty, and how did the case pack correct it?

## Evidence Log - Session 3

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 3 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 4: Performing Threat Using STRIDE

Total duration: 120 minutes

## Scenario

A selected Trainer Rack architecture segment is assumed to face a credible multi-stage threat. Your team builds a structured threat model of that segment: define what is being protected, decompose it into assets and data flows, use STRIDE to identify how it could be attacked, check which threats are already mitigated, and propose mitigations for the rest. This is a threat-modelling exercise. You are not producing scored risk ratings or a risk register: that is covered later in the course.

## Learning objectives

- Define a defensible threat-model scope: what is being protected, the assets, boundaries, data flows and trust levels.

- Decompose the segment into a simple data-flow view suitable for threat analysis.

- Apply STRIDE systematically to every asset and data flow, recording the threat, affected asset, attack vector and entry point.

- Identify which threats are already addressed by existing controls and which are not.

- Flag safety-relevant threats explicitly.

- Propose candidate mitigations mapped to STRIDE categories and architecture location.

- Use Generative AI to challenge the threat model and validate its suggestions against evidence.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Trainer Rack architecture segment | Reference architecture for a realistic threat model |
| Session 1 asset inventory and Purdue / zone mapping | Asset and data-flow starting point |
| Session 3 Defensive Control Mapping Matrix | Source of existing-control evidence |
| STRIDE method | Threat identification across S, T, R, I, D, E |
| Threat model worksheet (this section) | Scope, data-flow view, threat table, mitigation plan |
| Generative AI (<AI_TOOL>) | Challenge scope, missed threats and asset mappings; validate against evidence |

## Pre-flight checklist

☐ Trainer has selected the architecture segment in scope.

☐ Session 1 Asset Inventory / Purdue mapping and Session 3 Defensive Control Mapping Matrix to hand.

☐ STRIDE reference available.

☐ Access to <AI_TOOL> confirmed.

## Exercise steps

### Part A: Scope and decomposition

- The trainer selects the architecture segment. Record it, and state what the model is protecting: the security and safety objectives that matter for this segment.

- Document scope, assets, boundaries, trust levels and data flows for the segment.

Threat Model Scope

| Field | Detail |
| --- | --- |
| Architecture segment in scope |  |
| What we are protecting (objectives) |  |
| In-scope assets |  |
| Out-of-scope (incl. SIS island unless stated) |  |
| Trust boundaries |  |
| External entities and actors |  |
| Assumptions |  |

Assets and data flows in scope

| Asset / data flow | Purdue level / zone | Function | Talks to / carries | Crosses a trust boundary? (Y/N) | Existing controls |
| --- | --- | --- | --- | --- | --- |

- Draw a simple data-flow view of the segment: boxes for assets and actors, arrows for flows, dashed lines for trust boundaries. Attach it as evidence M1-W4-<KIT_ID>-03-dataflow.png.

⚠  A hand sketch or whiteboard photo is fine. The goal is to see every flow that crosses a trust boundary, because those are where most STRIDE threats live.

### Part B: STRIDE threat identification

- For every asset and every data flow, walk through S, T, R, I, D and E. Record each credible threat: the attacker action, the asset or flow affected, the attack vector, the plausible entry point, and whether it is safety-relevant.

STRIDE reference

| Letter | Category | OT example |
| --- | --- | --- |
| S | Spoofing | Rogue node impersonates the engineering workstation to the PLC |
| T | Tampering | Modified setpoint / logic download / firmware |
| R | Repudiation | Control action with no attributable log or operator identity |
| I | Information disclosure | Plaintext protocol reveals process values and addressing |
| D | Denial of service | Flood or malformed packets stall cyclic control traffic |
| E | Elevation of privilege | Gaining engineering rights from an operator or HMI foothold |

STRIDE Threat Table

| ID | STRIDE cat. | Threat (attacker action) | Affected asset / flow | Attack vector | Plausible entry point | Safety-relevant? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |

- Document attacker profiles relevant to this segment: who would realistically attempt these threats, and how capable they are.

Attacker Profiles

| Profile | Motivation | Capability | Likely objective in this segment | Plausible entry point |
| --- | --- | --- | --- | --- |

### Part C: Existing controls and gaps

- For each threat, record whether an existing control already addresses it (draw on Session 3), the control type, how effective it is, and whether a gap remains.

Threat / Control Mapping

| Threat ID | Existing control (if any) | Control type (prevent / detect / respond) | Effective? (Y / Partial / N) | Gap remaining |
| --- | --- | --- | --- | --- |

- Mark the threats with no effective control and the threats that are safety-relevant. These are the focus of the mitigation plan.

### Part D: Mitigation plan

- For each unmitigated or safety-relevant threat, propose one or more candidate mitigations. Map each to the STRIDE category it addresses and to where it sits in the architecture (host, conduit, zone boundary, monitoring, or process / procedure).

Mitigation Plan

| Threat ID | Candidate mitigation | STRIDE category addressed | Architecture location | Owner / next step |
| --- | --- | --- | --- | --- |

- Note any threat you are deliberately not mitigating now and why (for example: covered by SIS island isolation, out of scope, or requires a vendor change). This is a modelling note, not a formal risk-acceptance decision.

Deliberately-not-mitigated notes

| Threat ID | Reason not mitigated now | Revisit when |
| --- | --- | --- |

### Part E: Challenge the threat model with AI

- Use <AI_TOOL> to challenge the threat model. Ask it to find assets or data flows you missed, STRIDE categories you skipped for an asset, threats mapped to the wrong asset, and over-broad threats that are not credible here. Validate every suggestion against your architecture and your Session 1 to Session 3 evidence.

Suggested AI prompt

Here is a threat model for an OT architecture segment, including the scope, asset list, data-flow list, and STRIDE threat table. Act as a critical reviewer of the threat model. Identify the following: - Missing Assets or Flows – Assets or communication/data flows that appear necessary for the described architecture but are not included. - Missing STRIDE Categories – STRIDE categories that have not been considered for an asset where they are relevant to that asset or its flows. - Misattributed Threats – Threats assigned to the wrong asset or communication flow. - Non-Credible Threats – Threats in the table that are not credible or applicable given the stated architecture. For every finding, provide: - Asset / Flow: The specific asset or flow concerned. - Finding: What is missing, incorrect, or questionable. - Rationale: Why it is relevant or not credible based only on the architecture provided? - Evidence to Verify: The specific architecture detail, communication flow, configuration, or other evidence I should verify. Do not introduce generic threats or assumptions. Every finding must be traceable to a specific asset or communication flow provided in the architecture. If the available information is insufficient to determine whether something is missing, applicable, or credible, explicitly state the uncertainty rather than assuming it.

- Record each AI suggestion and your verdict.

AI Challenge Log

| AI suggestion | Type (missed asset / missed threat / wrong mapping / not credible) | Your evidence check | Verdict (accepted / rejected) |
| --- | --- | --- | --- |

- Update the STRIDE Threat Table and Mitigation Plan with any accepted additions.

### Deliverables: tick when complete and evidence is saved

☐ Threat Model Scope and assets / data-flows table

☐ Data-flow view of the segment (sketch or photo attached)

☐ STRIDE Threat Table covering every asset and data flow, safety-relevant threats flagged

☐ Attacker Profiles

☐ Threat / Control Mapping with gaps identified

☐ Mitigation Plan for unmitigated and safety-relevant threats

☐ Deliberately-not-mitigated notes

☐ AI Challenge Log with at least two suggestions validated or rejected against evidence

### Reflection questions

Which asset or data flow attracted the most STRIDE threats, and why?

Which threats are only 'mitigated' because of SIS island isolation? What happens to the model if that assumption is removed?

Where did applying STRIDE mechanically surface a threat you would otherwise have missed?

What did the AI review add that your team missed, and what did it get wrong?

### Evidence Log: Session 4

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |

| Trainee / Team | Trainer sign-off | Date | Evidence bundle ref |
| --- | --- | --- | --- |

# Module 1: Completion Record

To be signed once all four weekly evidence bundles are complete and stored at <EVIDENCE_STORE>.

| Session | Deliverables complete (Y/N) | Evidence hashed & stored | Trainer initials | Date |
| --- | --- | --- | --- | --- |
| Session 1 |  |  |  |  |
| Session 2 |  |  |  |  |
| Session 3 |  |  |  |  |
| Session 4 |  |  |  |  |

## Sign-Off

| Field | Signature / notes |
| --- | --- |
| Trainee / team lead signature |  |
| Trainer signature |  |
| Overall Module 1 outcome / notes |  |