Module 2

OT Asset and Configuration Management

Student Lab Workbook

Industrial Cybersecurity Training Program

| Trainee name |  |
| --- | --- |
| Team / Kit ID |  |
| Cohort / Class |  |
| Trainer |  |
| Start date |  |

# Contents

Contents	2

How to Use This Workbook	4

Working Method for Every Session	4

Evidence Naming Convention	4

Safety-First Rules	5

Kit Parameter Sheet	6

Module 2 Learning Outcomes	8

Module 2 Evidence Bundle (all four sessions)	8

Session 1: Passive Asset Discovery, Inventory Baseline and Asset Lifecycle Review	9

Scenario	9

Learning Objectives	9

What you will use this session	9

Pre-Flight Checklist	9

Exercise Steps	10

Part A - Passive Discovery from the Shared Capture	10

Part B - OT-Safe Active Discovery	11

Part C - Windows Host Evidence	12

Part D - Unified Inventory, Lifecycle and Confidence	14

Deliverables - tick when complete and evidence is saved	16

Reflection Questions	17

Evidence Log - Session 1	17

Session 2: Rogue Asset Detection, Baseline Deviation and Exposure Review	18

Scenario	18

Learning Objectives	18

What you will use this session	18

Pre-Flight Checklist	18

Exercise Steps	19

Part A - Import the Baseline	19

Part B - Re-Discover the Current State	19

Part C - Compare Against Baseline	20

Part D - Investigate Deviations	20

Part E - Classify, Dispose and Report	22

Deliverables - tick when complete and evidence is saved	23

Reflection Questions	23

Evidence Log - Session 2	24

Session 3: Hardening, Physical Security, USB and Supply-Chain Attacks	25

Scenario	25

Learning Objectives	25

What you will use this session	25

Pre-Flight Checklist	25

Exercise Steps	26

Part A - Pre-Hardening Baseline and Threat Primer	26

Part B – Malicious USB and Host Hardening	27

Part C - Rogue Network Device and Switch Hardening	28

Part D - Firmware Supply-Chain and Integrity	29

Part E - Policy and Anomaly Hunt	30

Deliverables - tick when complete and evidence is saved	31

Reflection Questions	31

Evidence Log - Session 3	32

Session 4: Patch, Change, Backup, Restore and Integrity Readiness	33

Scenario	33

Learning Objectives	33

What you will use this session	33

Pre-Flight Checklist	33

Exercise Steps	34

Part A - Scope and Backup	34

Part B - Patch Decision Framework	36

Part C - WSUS and Maintenance Schedule	38

Part D - Change Request, Rollback and Go / No-Go	38

Deliverables - tick when complete and evidence is saved	40

Reflection Questions	41

Evidence Log - Session 4	41

Module 2: Completion Record	43

Sign-Off	43

# How to Use This Workbook

Module 2 addresses the management of OT assets and their configuration across the system lifecycle, progressing over four 120-minute sessions from establishing an asset and lifecycle baseline, through detecting deviation from it, to hardening against physical, removable-media and supply-chain attack, and finally to controlled patch, change, backup and restore. This workbook is the structured record for that work: each session sets out the scenario, objectives, procedure and templates, and the completed inventories, registers, comparison tables, hardening records and integrity hashes together constitute the assessed evidence bundle for the module. Work is carried out by the team against the Trainee Kit, supported by Trainer Rack demonstrations and by shared services on the Central infrastructure: Splunk, SINEC NMS and SINEC Security Monitor, and WSUS with Active Directory.

Module 2 builds directly on Module 1. The Module 1 Asset Inventory, Purdue and zone mapping, and Communication Flow Matrix form the baseline that this module extends, defends and tests, and should be kept available throughout.

## Working Method for Every Session

- Read the scenario and objectives for the session before touching any tool.

- Complete the pre-flight checklist. Do not start the exercise until every item is ticked.

- Work through the numbered steps in order. Where a command, filter or prompt is given in a grey box, use it as written unless the trainer changes a placeholder value.

- Fill in the tables as you go, not from memory afterwards. Mark every entry as Observed, Inferred or Assumed.

- Capture evidence using the naming convention below and log it in the Evidence Log.

- Use Generative AI only where the step tells you to, and always validate its output against your own evidence: PCAP, Nmap output, Asset data, dashboards and vendor advisories. Record where the AI was wrong.

- Get the trainer sign-off at the end of the session.

## Evidence Naming Convention

M2-W<session>-<KIT_ID>-<step>-<short-description>.<ext> Examples: M2-W1-KIT03-04-nmap-discovery.txt M2-W2-KIT03-05-baseline-diff.xlsx M2-W3-KIT03-09-firmware-hash.png M2-W4-KIT03-09-change-request.docx

For any exported file, compute a SHA-256 hash and record it in the Evidence Log.

# Windows PowerShell: hash an evidence file Get-FileHash -Algorithm SHA256 '.\M2-W1-KIT03-04-nmap-discovery.txt'

## Safety-First Rules

- No configuration change, patch, hardening action or backup/restore without a written backup, a rollback plan and trainer approval. This module changes host, switch and firewall settings: every change is reversible and pre-approved.

- The SIS (Safety Instrumented System) island is out of scope for any scan, write, injection, hardening change or restore at all times. Observe it only through passive evidence and dashboards.

- OT-safe scanning only. Active discovery is limited to trainer-approved targets, with conservative timing. No aggressive scans, no exploit modules, no scripts against control devices. Stop if any device alarms or drops communication.

- Removable media and firmware samples used in this module are trainer-supplied and benign. Never connect personal USB devices to lab equipment; never flash firmware in Module 2 (hash-verify only).

- Generative AI is a challenger, not a source of truth. Never paste credentials, full internal IP schemes, or client-identifying data into an external AI tool. Every AI claim about a CVE, lifecycle date or advisory must be verified against the vendor source.

# Kit Parameter Sheet

Complete this table before Session 1 and refer back to it throughout the module.

Your trainer will give you the values for your kit, the Trainer Rack and the shared Central infrastructure. Use these placeholder names wherever the steps refer to them.

| Placeholder | Value (fill in) | Description | Notes |
| --- | --- | --- | --- |
| <KIT_ID> |  | Your kit / team identifier | Used in evidence file names |
| <PLC_IP> |  | PCS 7 CPU 410 controller IP | Control zone |
| <ET200SP_IP> |  | ET 200SP I/O station IP | Control zone |
| <HMI_WS_IP> |  | Operator / HMI workstation IP |  |
| <ENG_STATION_IP> |  | Engineering station IP | Backup / project source |
| <SC646_MGMT_IP> |  | SCALANCE SC646-2C firewall mgmt IP | Zone boundary device |
| <XC208_MGMT_IP> |  | SCALANCE XC208G switch mgmt IP | Access switch |
| <KIT_SUBNET> |  | Kit subnet / mask (e.g. 192.168.10.0/24) | Discovery scope |
| <KIT_GW> |  | Kit default gateway |  |
| <OT_VLAN> |  | OT / control VLAN ID |  |
| <SUPPORT_VLAN> |  | Maintenance / engineering VLAN ID |  |
| <SCAN_HOST> |  | Host running Nmap (workstation or Kali) | Trainer-approved only |
| <APPROVED_TARGETS> |  | IP / range approved for active discovery | Excludes SIS + fragile devices |
| <TR_PCAP_NAME> |  | Shared Trainer Rack PCAP file name | Passive discovery source |
| <TR_SIS_SEG> |  | Trainer Rack SIS island segment / VLAN | OUT OF SCOPE for active work |
| <SINEC_NMS_URL> |  | SINEC NMS dashboard URL | Central infrastructure |
| <SINEC_SSM_URL> |  | SINEC Security Monitor URL | Central infrastructure |
| <SPLUNK_URL> |  | Splunk web URL | USB / event evidence |
| <WSUS_SERVER> |  | WSUS server name / URL | Session 4 |
| <AD_DOMAIN> |  | Active Directory domain / OU for OT hosts | Session 4 |
| <BACKUP_STORE> |  | Path / share for backups and exports | Session 4 |
| <FW_IMAGE_GOOD> |  | Legitimate firmware sample file name + published hash | Session 3, hash-verify only |
| <FW_IMAGE_TEST> |  | Tampered firmware sample file name | Session 3, hash-verify only |
| <AI_TOOL> |  | Approved Generative AI tool / URL | Validation assistant only |
| <EVIDENCE_STORE> |  | Path / share for the evidence bundle |  |

# Module 2 Learning Outcomes

By the end of Module 2 you will be able to:

- Establish a defensible OT asset baseline using passive capture, OT-safe active discovery and Windows host evidence, and state your confidence in each entry.

- Maintain an asset lifecycle register covering support status, firmware version, and end-of-service / end-of-life exposure.

- Detect deviation from a baseline and classify rogue, undocumented, maintenance and unresolved changes using discovery and monitoring evidence.

- Apply and verify host, network, physical and supply-chain hardening controls against BadUSB, rogue-device and tampered-firmware scenarios.

- Verify firmware and file integrity with SHA-256 and interpret a mismatch correctly.

- Plan and document backup, restore, patch and change activities for OT, including a controlled rollback path and a go / no-go decision.

- Use Generative AI to accelerate correlation and drafting, then validate every output against packet, host, dashboard and vendor evidence.

## Module 2 Evidence Bundle (all four sessions)

| Session | Required outputs |
| --- | --- |
| Session 1 | Passive Discovery Findings; Nmap Discovery Findings; Windows Host Evidence; Unified Asset Inventory; Asset Lifecycle Register; Discovery Confidence Report; AI Validation Log |
| Session 2 | Baseline Import Checklist; Re-Discovery Results; Baseline vs Current Diff; Deviation Investigation Log; SINEC Dashboard Observations; Deviation Classification & Disposition; Asset Deviation Summary Report |
| Session 3 | Pre-Hardening Config Baseline; Threat Primer Quiz; Physical Walk-Down Findings; Host Hardening Actions & Verification; Rogue Device Detection Log; Switch Hardening Actions; Firmware Pre-Flash Checklist; Firmware Hash Verification; OT Physical & Removable Media Policy (draft); Spot-the-Plant Log |
| Session 4 | Maintenance & Backup Scope; Backup Checklists; Backup Register with Metadata; Integrity Hash Record; OT Patch Decision Framework; Patch Decision Log; AI vs Vendor/OT Review; WSUS Deployment & Maintenance Schedule; Change Request; Rollback Plan; Change Package Completeness Review; Go / No-Go Decision |

# Session 1: Passive Asset Discovery, Inventory Baseline and Asset Lifecycle Review

Total duration: 120 minutes

## Scenario

The organization has incomplete visibility of its OT environment and cannot confirm whether all connected systems are documented or supported. Your team must establish a defensible asset and lifecycle baseline using passive, active, host and vendor evidence, and state how confident you are in each entry.

## Learning Objectives

- Extract assets, protocols, IPs, MACs and communication relationships from a passive capture.

- Run OT-safe active discovery with Nmap against trainer-approved targets only.

- Collect and interpret Windows host inventory evidence with PowerShell.

- Merge passive, active, host and AI-assisted evidence into one unified asset profile per device.

- Record firmware / support status and EoS / EoL exposure in a lifecycle register.

- Produce a discovery confidence report distinguishing confirmed assets from assumptions.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Wireshark / PCAP | Passive discovery of assets, protocols, IPs, MACs and communication relationships |
| Nmap | OT-safe active discovery of hosts, ports, operating systems and services |
| PowerShell | Windows host information collection and inventory enrichment |
| Asset Inventory / Lifecycle Register | Baseline documentation, support-status tracking and discovery-confidence reporting |
| Generative AI (<AI_TOOL>) | Correlate discovery evidence and classify lifecycle / legacy-asset constraints |

## Pre-Flight Checklist

☐ Module 1 Asset Inventory and Purdue/zone mapping to hand.

☐ Kit Parameter Sheet completed, including <APPROVED_TARGETS> and <SCAN_HOST>.

☐ Shared passive PCAP <TR_PCAP_NAME> received and opens in Wireshark.

☐ Nmap installed on <SCAN_HOST>; you can reach <APPROVED_TARGETS> and NOT the SIS segment.

☐  PowerShell available on the Windows workstation with rights to run inventory commands.

☐  Trainer has confirmed the approved active-discovery scope and timing profile.

☐  Access to <AI_TOOL> confirmed; evidence folder created at <EVIDENCE_STORE>.

## Exercise Steps

### Part A - Passive Discovery from the Shared Capture

- The trainer identifies passive capture locations on the Trainer Rack and distributes the shared PCAP <TR_PCAP_NAME>. Open it in Wireshark.

- Analyze the capture for assets, protocols, IP addresses, MAC addresses and communication relationships. Useful views and filters are given below.

Statistics > Endpoints (IPv4 + Ethernet; note MAC OUI vendor) Statistics > Conversations (who talks to whom; ports; duration) Statistics > Protocol Hierarchy (protocol mix) Discovery-oriented display filters: arp address resolution / who-has bootp ¦¦ dhcp host + option 12 / 60 (name, vendor class) mdns ¦¦ llmnr ¦¦ nbns name broadcasts reveal hostnames cdp ¦¦ lldp switch / device neighbor announcements browser Windows browser elections (workgroup, roles) s7comm ¦¦ cotp PCS 7 / S7 controllers modbus ¦¦ dnp3 ¦¦ iec60870_104 other OT endpoints

- Record every device seen in the Passive Discovery Findings table. Mark each as Observed.

Table W1.1 - Passive Discovery Findings

| IP address | MAC / OUI vendor | Hostname (if seen) | Protocols seen | Talks to | Likely role | Evidence (frame/stat) |
| --- | --- | --- | --- | --- | --- | --- |

- Ask <AI_TOOL> to extract an asset list from a pasted summary of your Wireshark Endpoints / Protocol Hierarchy output. Compare the AI list against the devices you identified manually. Record matches, misses and anything the AI invented.

#### Suggested AI prompt

Here is a summary of endpoints and protocols identified from a passive OT network capture. Based only on the provided capture data, identify the distinct assets that can be reasonably inferred. For each asset, provide: - Asset: IP address, hostname, MAC address, or another available identifier. - Likely Device Type: For example, PLC, HMI, engineering workstation, SCADA server, network device, or other device class. - Evidence: The specific endpoint, protocol, port, communication pattern, or other observed characteristic supporting the inference. - Confidence: High / Medium / Low. - Inference Flag: Clearly identify any device type or role that is inferred rather than directly supported by the capture. Do not invent or assume assets that are not supported by the provided data. Do not treat an IP address, protocol, or port alone as definitive proof of a device type unless the evidence supports that conclusion. Where the available evidence is insufficient to determine the device type or role, state “Unknown” and explain what additional evidence would be required to identify it.

### Part B - OT-Safe Active Discovery

⚠ SAFETY Active discovery in OT can disrupt fragile devices. Scan only <APPROVED_TARGETS>. Never scan the SIS segment, the RTU or the relay unless the trainer has explicitly listed them. Use conservative timing and stop immediately if a device alarms.

- Run a host-discovery sweep, then a light service scan, against <APPROVED_TARGETS> only. Save all output.

# Host discovery only (no port scan) nmap -sn --disable-arp-ping <APPROVED_TARGETS> -oA M2-W1-<KIT_ID>-03-hostsweep # Light, slow TCP scan of common OT/IT ports on approved hosts nmap -sT -Pn -T2 --scan-delay 200ms --max-retries 1 \ -p 22,23,80,102,161,443,445,502,2404,20000,44818,34962-34964 \ <APPROVED_TARGETS> -oA M2-W1-<KIT_ID>-04-portscan # AVOID in OT unless trainer-approved: # -O -A -sV --version-intensity>2 -sU sweeps NSE scripts

- Record the Nmap Discovery Findings: host, open ports, service guesses, and whether this matches the passive evidence.

Table W1.2 - Nmap Discovery Findings

| Host IP | State | Open ports | Service / product (guess) | Matches passive evidence? (Y/N) | Notes / anomalies |
| --- | --- | --- | --- | --- | --- |

### Part C - Windows Host Evidence

- On the Windows workstation (and any other approved Windows host), collect host inventory with PowerShell and save the output.

# Identity + OS Get-ComputerInfo ¦ Select CsName,OsName,OsVersion,OsBuildNumber,WindowsProductName,CsDomain # Network Get-NetIPConfiguration Get-NetIPAddress -AddressFamily IPv4 ¦ Select IPAddress,InterfaceAlias # Software + patch level Get-HotFix ¦ Sort InstalledOn -Descending ¦ Select -First 15 Get-CimInstance Win32_Product ¦ Select Name,Version,Vendor ¦ Sort Name # Services + autoruns + local users Get-Service ¦ Where Status -eq 'Running' ¦ Select Name,DisplayName,StartType Get-CimInstance Win32_StartupCommand ¦ Select Name,Command,Location Get-LocalUser ¦ Select Name,Enabled,LastLogon # USB history (feeds Session 3) Get-ItemProperty 'HKLM:\SYSTEM\CurrentControlSet\Enum\USBSTOR\*\*' -EA SilentlyContinue ¦ Select FriendlyName,PSChildName

- Complete the Windows Host Evidence table for each host.

Table W1.3 - Windows Host Evidence

| Hostname | OS / build | Domain / workgroup | IP(s) | Last patch (KB / date) | Notable services / software | Local admin accounts |
| --- | --- | --- | --- | --- | --- | --- |

- Ask <AI_TOOL> to correlate the passive, active and host evidence into one profile per asset. Validate every merged field against a specific source before accepting it.

#### Suggested AI prompt

I will provide Wireshark (passive), Nmap (active), and PowerShell (host-level) evidence for the same OT lab. Create one consolidated record per asset with: IP ¦ MAC ¦ Hostname ¦ Role ¦ OS/Firmware ¦ Open Services ¦ Peers For every field, cite the source: Wireshark / Nmap / PowerShell. - Mark unsupported fields as “Unverified.” - Do not infer unsupported details. - Flag conflicting evidence between sources. - Clearly distinguish observed facts from inferences.

### Part D - Unified Inventory, Lifecycle and Confidence

- Build the Unified Asset Inventory by merging all three evidence streams plus your Module 1 inventory.

Table W1.4 - Unified Asset Inventory

| # | Asset / model | IP | MAC | Role | Purdue / zone | Firmware / OS version | Evidence sources (P/A/H/M1) | Confidence (H/M/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |

- Research vendor support status, firmware lifecycle, and EoS / EoL dates and advisories. Use <AI_TOOL> to speed up the search, then validate every date and advisory ID against the vendor page (Siemens ProductCERT / support portal, or the relevant vendor).

- Use <AI_TOOL> to classify legacy assets, identify operational constraints, recommend candidate controls, and rate confidence. Record only what you can support.

Table W1.5 - Asset Lifecycle Register

| Asset | Current firmware / OS | Latest available | Support status (supported / extended / EoS / EoL) | Key advisory / CVE (verified source) | Constraint if replaced / patched | Candidate control |
| --- | --- | --- | --- | --- | --- | --- |

- Complete the Discovery Confidence Report: for each asset, what is confirmed, what is assumed, and what evidence would raise confidence.

Table W1.6 - Discovery Confidence Report

| Asset | Confirmed facts | Assumptions / gaps | Evidence needed to confirm | Overall confidence (H/M/L) |
| --- | --- | --- | --- | --- |

Table W1.7 - AI Validation Log

| Where AI was used | AI output | Your evidence check | Verdict (accepted / corrected / rejected) |
| --- | --- | --- | --- |

## Deliverables - tick when complete and evidence is saved

☐  Passive Discovery Findings from <TR_PCAP_NAME>

☐  Nmap Discovery Findings (approved targets only; raw -oA output saved)

☐  Windows Host Evidence for each approved host

☐  Unified Asset Inventory with evidence-source and confidence columns

☐  Asset Lifecycle Register with vendor-verified support status / advisories

☐  Discovery Confidence Report

☐  AI Validation Log with at least two AI errors or unverified claims documented

☐  Evidence Log updated; files hashed

## Reflection Questions

Which asset had the weakest evidence, and what single collection step would fix that?

Did active discovery reveal anything passive capture missed, or vice versa? Why?

Where did the AI's lifecycle information disagree with the vendor page, and how did you resolve it?

## Evidence Log - Session 1

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 1 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 2: Rogue Asset Detection, Baseline Deviation and Exposure Review

Total duration: 120 minutes

## Scenario

A rogue or unauthorized system, service or configuration deviation appears after the Session 1 baseline was established. Your team must detect, investigate, classify and report the change using the available discovery and monitoring evidence.

## Learning Objectives

- Re-run discovery and compare the current state against a stored baseline.

- Identify new, missing, modified and undocumented assets and services.

- Investigate a deviation using packet, host, scan and dashboard evidence together.

- Use SINEC NMS / SINEC Security Monitor to confirm or refute a suspected rogue asset.

- Classify each deviation and assign a disposition with a documented rationale.

- Produce an Asset Deviation Summary Report suitable for an operations handover.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Session 1 Baseline Inventory | Reference point for deviation and rogue-asset analysis |
| Wireshark | Traffic evidence collection and comparison against baseline |
| Nmap | Current-state discovery of new or changed hosts and services |
| PowerShell | Host evidence collection for Windows-based systems |
| SINEC NMS / SINEC Security Monitor | Deviation investigation, visibility validation, rogue-asset confirmation |
| Generative AI (<AI_TOOL>) | Correlate baseline / current evidence and assess significance of deviations |

## Pre-Flight Checklist

☐  Session 1 Unified Asset Inventory, host evidence and raw Nmap output available and unmodified.

☐  Kit Parameter Sheet to hand; <APPROVED_TARGETS> unchanged from Session 1.

☐  Access to <SINEC_NMS_URL> and <SINEC_SSM_URL> confirmed (Central infrastructure).

☐  Discovery tools verified on <SCAN_HOST> and the workstation.

☐  Trainer has confirmed the deviation scenario window is active.

☐  Access to <AI_TOOL> confirmed.

## Exercise Steps

### Part A - Import the Baseline

- Import the Session 1 baseline inventory and host evidence. Do not edit the baseline files: work on copies.

Table W2.1 - Baseline Import Checklist

| Baseline artefact | Present? (Y/N) | Hash matches Session 1 log? |
| --- | --- | --- |
| Unified Asset Inventory (Session 1) |  |  |
| Windows Host Evidence (Session 1) |  |  |
| Nmap raw output (Session 1) |  |  |
| Passive Discovery Findings (Session 1) |  |  |

### Part B - Re-Discover the Current State

- The trainer introduces one or more rogue / unauthorized systems on the Trainer Rack (and possibly your kit). Repeat discovery using the same methods and scope as Session 1: Wireshark, Nmap, PowerShell.

# Re-capture a short passive sample on the same monitor point, then: nmap -sn -PR <APPROVED_TARGETS> -oA M2-W2-<KIT_ID>-03-hostsweep nmap -sT -Pn -T2 --scan-delay 200ms \ -p 22,23,80,102,161,443,445,502,2404,20000,44818 \ <APPROVED_TARGETS> -oA M2-W2-<KIT_ID>-04-portscan # Compare port scans quickly (if ndiff is available): ndiff M2-W1-<KIT_ID>-04-portscan.xml M2-W2-<KIT_ID>-04-portscan.xml

- Consolidate the new evidence and record the Re-Discovery Results.

Table W2.2 - Re-Discovery Results

| IP / MAC | Hostname / role | Open ports / services now | In Session 1 baseline? (Y/N) | Evidence source(s) |
| --- | --- | --- | --- | --- |

### Part C - Compare Against Baseline

- Compare current results against the Session 1 baseline and classify every difference as New, Missing, Modified, or Undocumented.

Table W2.3 - Baseline vs Current Diff

| Asset / service | Change type (New / Missing / Modified / Undocumented) | Session 1 value | Current value | First evidence of change |
| --- | --- | --- | --- | --- |

### Part D - Investigate Deviations

- For each significant deviation, investigate using Wireshark, Nmap, PowerShell and the SINEC NMS / SINEC Security Monitor dashboards together. Record what each source shows.

Table W2.4 - Deviation Investigation Log

| Deviation | Wireshark evidence | Nmap / host evidence | SINEC NMS / SSM evidence | Working hypothesis |
| --- | --- | --- | --- | --- |

- Record specific SINEC Dashboard Observations: is the device visible to SINEC, when did it first appear, what alerts or anomalies are raised, and what asset context is shown.

Table W2.5 - SINEC Dashboard Observations

| Device / deviation | Visible in SINEC? (NMS / SSM) | First seen timestamp | Alerts / anomaly type | Asset context shown |
| --- | --- | --- | --- | --- |

- Use <AI_TOOL> to correlate the baseline and current results and explain the discrepancies, then to evaluate the significance of each deviation. Validate every AI conclusion against the evidence in your logs.

#### Suggested AI prompt

Compare the baseline and current asset lists for the same OT segment, using the dashboard notes as supporting context. For each difference, provide: Difference ¦ Most likely explanation ¦ Operational significance (Low/Med/High) ¦ Evidence to confirm - Do not assume malicious intent without supporting evidence. - Base explanations only on the provided data. - Clearly flag any uncertain inference.

### Part E - Classify, Dispose and Report

- Validate the AI findings and classify each deviation as Rogue, Legitimate but Undocumented, Temporary Maintenance Activity, or Unresolved. Assign a disposition and owner.

Table W2.6 - Deviation Classification & Disposition

| Deviation | Classification | Rationale (evidence) | Disposition / action | Owner | Due |
| --- | --- | --- | --- | --- | --- |

- Prepare the Asset Deviation Summary Report (one page): what changed, how it was detected, what it means operationally, and the recommended actions.

Table W2.7 - Asset Deviation Summary Report

| Section | Content |
| --- | --- |
| Summary of changes |  |
| Detection method & timeline |  |
| Operational significance |  |
| Confirmed rogue assets |  |
| Recommended actions & owners |  |

## Deliverables - tick when complete and evidence is saved

☐  Baseline Import Checklist with hash verification

☐  Re-Discovery Results (raw Nmap output saved; passive sample saved)

☐  Baseline vs Current Diff with every change classified

☐  Deviation Investigation Log

☐  SINEC Dashboard Observations

☐  Deviation Classification & Disposition with owners

☐  Asset Deviation Summary Report (one page)

☐  AI correlation output captured; at least one AI claim corrected against evidence

## Reflection Questions

Which single source first revealed the rogue asset, and would it have been caught without SINEC?

Was any real deviation hidden by a benign one? How did you separate them?

If this were production, what monitoring change would shorten time-to-detection?

## Evidence Log - Session 2

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 2 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 3: Hardening, Physical Security, USB and Supply-Chain Attacks

Total duration: 120 minutes

## Scenario

The environment is exposed to three controlled attack vectors: a benign BadUSB event, connection of a rogue network device, and a tampered firmware sample. Your team must apply and verify host, network, physical and supply-chain controls, and confirm each control actually works.

## Learning Objectives

- Capture a pre-hardening configuration baseline you can roll back to.

- Identify physical and removable-media attack surfaces in a walk-down.

- Configure and verify USB storage blocking,and USB insertion logging to Splunk.

- Enforce switch port security and confirm a rogue network device is blocked and detected.

- Run a firmware supply-chain check and verify firmware integrity with SHA-256.

- Draft an enforceable OT Physical and Removable Media Policy.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| SCALANCE SC646-2C | Firewall hardening, port / security-control verification, rogue-device blocking |
| SCALANCE XC208G | Switch hardening, unused-port shutdown, MAC-limit enforcement |
| Workstation / Windows controls | USB blocking, host hardening checks |
| Splunk | USB insertion logging and event-evidence collection |
| SINEC NMS | Rogue-device visibility and network-anomaly confirmation |
| Hashing utility / PowerShell | Firmware integrity validation for legitimate and tampered samples |
| Trellix EPO | Host Antivirus and Application/Hardware Allowlisting/Blocklisting Solution |

## Pre-Flight Checklist

☐  Trainer approval to change host, switch and firewall settings, with a rollback plan agreed.

☐  Config backup method ready for SC646-2C, XC208G and the workstation.

☐  Trainer-supplied benign USB device and firmware samples <FW_IMAGE_GOOD> / <FW_IMAGE_TEST> available (hash-verify only, no flashing).

☐  Access to <SPLUNK_URL> and <SINEC_NMS_URL> confirmed.

☐  Published hash for <FW_IMAGE_GOOD> obtained from the vendor page.

☐  You know which switch ports are in use and which are spare.

## Exercise Steps

### Part A - Pre-Hardening Baseline and Threat Primer

- Capture the pre-hardening configuration of SC646-2C, XC208G and the workstation. Store each as a named rollback file and hash it.

Table W3.1 - Pre-Hardening Config Baseline

| Device | Config export file | Method | SHA-256 (first 12) |
| --- | --- | --- | --- |
| SCALANCE SC646-2C |  |  |  |
| SCALANCE XC208G |  |  |  |
| Windows workstation |  |  |  |

- Complete the incident-vector threat primer quiz below. Answer from what you know, then check with the trainer.

Table W3.2 - Threat Primer Quiz

| # | Question | Your answer |
| --- | --- | --- |
| 1 | What makes a BadUSB device different from a USB storage-borne malware infection? |  |
| 3 | Name two switch features that limit an unauthorised device plugged into a live port. |  |
| 4 | What is the difference between authenticity and integrity for a firmware image? |  |
| 5 | Give two supply-chain checks that should happen before any OT firmware is flashed. |  |
| 6 | Why is 'air-gapped' not a sufficient control against removable-media threats? |  |

- Conduct a physical walk-down of the kit and rack area. Document exposed ports, unlocked enclosures, accessible USB ports, cabling and labelling issues.

Table W3.3 - Physical Walk-Down Findings

| Location / asset | Attack surface observed | Who could reach it | Risk (H/M/L) | Recommended control |
| --- | --- | --- | --- | --- |

### Part B – Malicious USB and Host Hardening

- Observe the trainer-led Malicious USB demonstration on the Trainer Rack (benign payload). Record what the payload did and what evidence it left.

- Observe trainer performing policy changes and logging methodology using Trellix Solution

Table W3.4 – Malicious USB Demo Observations

| Actions observed | Time to execute | Evidence left (host / Splunk) | What would have stopped it |
| --- | --- | --- | --- |

- On the workstation, configure USB storage blocking, and USB insertion logging to Splunk. Then test each control with the trainer-supplied device.

# USB mass-storage: Observer trainer performing the steps in Trellix UI # Or via Group Policy (preferred, reversible, audited): # Computer Config > Admin Templates > System > Removable Storage Access # 'All Removable Storage classes: Deny all access' = Enabled # 'Removable Disks: Deny write/execute' as needed # Verify USB events reach Splunk (examples): # index=* source=WinEventLog:Security EventCode=6416 (PnP device) # index=* sourcetype=XmlWinEventLog EventCode IN (2003,2100,2102) (USB Hub/Storage) # index=* sourcetype=Sysmon EventCode=6 (driver load, if used)

Table W3.5 - Host Hardening Actions & Verification

| Control | How applied (GPO / registry/ Trellix) | Rollback reference | Test performed | Result (blocked / logged / bypassed) |
| --- | --- | --- | --- | --- |

### Part C - Rogue Network Device and Switch Hardening

- Insert the trainer-supplied rogue network device on a live or spare port. Observe detection in SINEC NMS and on the SC646-2C. Record what each showed and how fast.

Table W3.6 - Rogue Device Detection Log

| Port used | Device presented as (MAC / OUI) | Detected by SINEC NMS? (time) | Seen on SC646-2C? (log line) | Reached anything? (evidence) |
| --- | --- | --- | --- | --- |

- Enable port security / MAC limits, disable unused ports, and re-test. Confirm the rogue device is now blocked.

# SCALANCE XC208G hardening (concept - use the web UI or CLI as configured): # - Limit MAC addresses per access port to 1 (or the known count) # - Set violation action to 'drop/shutdown' and enable logging # - Administratively disable every unused port # - Disable unused management protocols (Telnet, HTTP) - keep HTTPS/SSH # - Confirm change is saved to the startup config

Table W3.7 - Switch Hardening Actions

| Action | Ports affected | Before | After | Re-test result | Rollback reference |
| --- | --- | --- | --- | --- | --- |

### Part D - Firmware Supply-Chain and Integrity

- Run the firmware supply-chain tabletop. List the checks that must pass before any OT firmware is flashed (source, authenticity, integrity, change approval, rollback / recovery readiness, maintenance window).

Table W3.8 - Firmware Supply-Chain Pre-Flash Checklist

| # | Check | How it is satisfied | Evidence required |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |

- Verify the legitimate and tampered firmware hashes against the published vendor hash. Interpret the result: a mismatch means do not use, and it does not by itself tell you whether the change was malicious or accidental.

Get-FileHash -Algorithm SHA256 '<FW_IMAGE_GOOD>' # compare to vendor-published value Get-FileHash -Algorithm SHA256 '<FW_IMAGE_TEST>' # expect: DOES NOT MATCH

Table W3.9 - Firmware Hash Verification

| File | Computed SHA-256 | Matches vendor hash? | Decision |
| --- | --- | --- | --- |
| <FW_IMAGE_GOOD> |  |  |  |
| <FW_IMAGE_TEST> |  |  |  |
| Vendor-published hash |  | n/a | reference |

### Part E - Policy and Anomaly Hunt

- Draft the OT Physical and Removable Media Policy using the template below. Keep every requirement testable.

Table W3.10 - OT Physical and Removable Media Policy (draft)

| Clause | Requirement | Applies to | How compliance is evidenced |
| --- | --- | --- | --- |
| Purpose & scope |  |  |  |
| Approved media only & registration |  |  |  |
| Scanning & transfer station |  |  |  |
| USB port control & exceptions |  |  |  |
| HID device control |  |  |  |
| Enclosure & port physical security |  |  |  |
| Visitor & contractor devices |  |  |  |
| Logging & review |  |  |  |
| Sanctions |  |  |  |

- Complete the Spot-the-Plant anomaly hunt: the trainer has hidden anomalies in the environment. Log each one, the evidence, and the control that should have prevented it.

Table W3.11 - Spot-the-Plant Anomaly Log

| # | Anomaly found | Where / how spotted | Evidence | Control that should prevent it |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

- Restore every device to its pre-hardening baseline OR confirm with the trainer which hardening changes are kept. Record the final state and hash the post-session configs.

## Deliverables - tick when complete and evidence is saved

☐  Pre-Hardening Config Baseline (3 devices) with hashes and rollback files

☐  Threat Primer Quiz completed

☐  Physical Walk-Down Findings

☐  Malicious USB Demo Observations

☐  Host Hardening Actions & Verification (block + log both tested)

☐  Rogue Device Detection Log + Switch Hardening Actions with re-test results

☐  Firmware Pre-Flash Checklist and Firmware Hash Verification

☐  OT Physical and Removable Media Policy draft

☐  Spot-the-Plant Anomaly Log

☐  Final device state recorded; configs restored or changes confirmed; Evidence Log updated

## Reflection Questions

Which control gave the largest risk reduction for the least operational disruption?

Which of your controls could actually address a Malicious USB device?

A firmware hash mismatch was found. What are the next three steps, and who decides?

## Evidence Log - Session 3

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 3 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Session 4: Patch, Change, Backup, Restore and Integrity Readiness

Total duration: 120 minutes

## Scenario

A maintenance change is proposed using a suspect patch package and incomplete backup evidence. Your team must determine whether the change may proceed, verify integrity and recovery readiness, and prepare a controlled rollback path, ending with a documented go / no-go decision.

## Learning Objectives

- Define maintenance scope and the matching backup scope for OT assets.

- Perform and document backups with the metadata needed to restore them.

- Verify integrity of backups, exports, firmware and patch files with SHA-256.

- Build and apply an OT patch decision framework that respects operational constraints.

- Plan a WSUS deployment and maintenance window using WSUS and Active Directory.

- Prepare a formal change request, rollback plan and change-package completeness review.

- Use Generative AI for decision support and completeness checks, then verify against vendor and OT requirements.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Engineering station | Backup, export, recovery evidence and maintenance-scope validation |
| SCALANCE firewall | Configuration backup, baseline export and rollback planning |
| Windows OT workstation | Patch readiness, backup metadata and integrity-record collection |
| Hashing utility / PowerShell | SHA-256 integrity checks for backups, exports, firmware and patch files |
| WSUS / Active Directory | Patch deployment planning and maintenance-window scheduling |
| Generative AI (<AI_TOOL>) | Patch decision support and change-package review for missing evidence |
| <BACKUP_TOOL> | Enterprise data protection and recovery software |

## Pre-Flight Checklist

☐  Trainer approval for backup / restore activity, with target systems agreed.

☐  Backup media / share <BACKUP_STORE> available and writable.

☐  Access to <WSUS_SERVER> and the <AD_DOMAIN> management view confirmed (Central infrastructure).

☐  Suspect patch package and its claimed hash supplied by the trainer.

☐  Module 2 Asset Lifecycle Register (Session 1) and hardening baselines (Session 3) to hand.

☐  Access to <AI_TOOL> confirmed.

## Exercise Steps

### Part A - Scope and Backup

- Identify the systems selected for maintenance and determine the backup scope: what must be captured to fully restore each system (config, project, data, licences, keys). Identify what kind of tooling is required to capture the Backup. E.g. <BACKUP_TOOL>, VM snapshots, Config downloads etc.

Table W4.1 - Maintenance Scope & Backup Scope

| System | Proposed change | Must back up (items) | Restore dependencies | Downtime window |
| --- | --- | --- | --- | --- |

- Create the backup checklist for the Engineering Station, SCALANCE Firewall and Windows OT Workstation.

Table W4.2 - Backup Checklist

| System | Item to back up | Method / tool | Location | Done? (Y/N) |
| --- | --- | --- | --- | --- |

- After the Trainer Rack backup demonstration, perform the Trainee Kit backups and record full metadata for each.

Table W4.3 - Backup Register with Metadata

| Backup file | Source system | Type (full / config / project) | Date / time | Operator | Size | SHA-256 (first 12) | Restore-tested? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Generate SHA-256 hashes for every backup file, export, firmware image and patch file. Record them in the Integrity Hash Record.

Get-ChildItem '<BACKUP_STORE>' -File -Recurse ¦ Get-FileHash -Algorithm SHA256 ¦ Select Hash,Path ¦ Export-Csv M2-W4-<KIT_ID>-04-integrity.csv -NoTypeInformation # Verify the suspect patch package against the trainer-supplied claimed hash: Get-FileHash -Algorithm SHA256 '<patch package>' # compare exactly, character for character

Table W4.4 - Integrity Hash Record

| File | Type | SHA-256 | Claimed / expected hash | Match? (Y/N) |
| --- | --- | --- | --- | --- |

### Part B - Patch Decision Framework

- Develop an AI-assisted OT Patch Decision Framework: the criteria and thresholds you will use to decide Patch / Defer / Compensate / Monitor for an OT asset. Use <AI_TOOL> to draft it, then adjust for OT reality (availability, safety, vendor validation, maintenance windows).

#### Suggested AI prompt

Draft a decision framework for determining whether to apply a security patch to an OT asset. Consider these inputs: - Asset criticality - Safety relevance - Network exposure - CVE severity - Exploit status - Vendor validation status - Rollback readiness - Maintenance-window availability Output one decision: Patch / Defer / Compensate / Monitor For each decision, provide: - Reasoning - Key factors driving the decision - Conditions or evidence that would change the decision Do not assume the patch is safe to deploy or that the vulnerability is exploitable. Flag any assumptions and uncertainties so I can validate them against vendor guidance and OT operational constraints.

Table W4.5 - OT Patch Decision Framework

| Criterion | How measured | Weight / threshold | Notes for OT |
| --- | --- | --- | --- |

- Apply the framework to the OT asset scenarios provided by the trainer. Record each decision and its rationale.

Table W4.6 - Patch Decision Log

| Asset / patch | CVE / severity | Exposure in lab | Vendor validated? | Decision (Patch / Defer / Compensate / Monitor) | Rationale |
| --- | --- | --- | --- | --- | --- |

- Critically review the AI recommendations against vendor and OT requirements. Record every place the AI over-simplified or missed an OT constraint.

Table W4.7 - AI vs Vendor / OT Review

| AI recommendation | Vendor / OT requirement | Gap | Corrected decision |
| --- | --- | --- | --- |

### Part C - WSUS and Maintenance Schedule

- Using Central infrastructure, develop the WSUS deployment and maintenance schedule with WSUS and Active Directory: target groups, approval flow, ring order, and the maintenance window.

Table W4.8 - WSUS Deployment & Maintenance Schedule

| Target group (AD OU / WSUS group) | Systems | Approval owner | Test ring / order | Maintenance window | Rollback method |
| --- | --- | --- | --- | --- | --- |

### Part D - Change Request, Rollback and Go / No-Go

- Prepare a formal Change Request and a Rollback Plan.

Table W4.9 - Change Request

| Field | Detail |
| --- | --- |
| Change ID |  |
| Title |  |
| Systems affected |  |
| Description |  |
| Reason |  |
| Risk & impact |  |
| Backout risk |  |
| Pre-checks |  |
| Implementation steps |  |
| Verification steps |  |
| Window |  |
| Approver |  |
| Requested by |  |
| Date |  |

Table W4.10 - Rollback Plan

| Step | Trigger to roll back | Rollback action | Artefact used (backup / hash ref) | Verification after rollback | Owner |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

- Use <AI_TOOL> to review the change package for missing evidence, then verify manually: backups present and hashed, integrity records complete, recovery evidence, approvals, rollback procedures.

Table W4.11 - Change Package Completeness Review

| Required element | Present? (Y / Partial / N) | Evidence reference | Gap / action |
| --- | --- | --- | --- |
| Maintenance scope |  |  |  |
| Backup of every affected system |  |  |  |
| Integrity hashes |  |  |  |
| Restore test |  |  |  |
| Patch decision & rationale |  |  |  |
| Vendor validation status |  |  |  |
| WSUS plan |  |  |  |
| Maintenance window |  |  |  |
| Change request |  |  |  |
| Rollback plan |  |  |  |
| Approvals |  |  |  |

- Record the Go / No-Go decision for the proposed change, with the conditions attached to a 'go' or the blockers behind a 'no-go'.

Table W4.12 - Go / No-Go Decision

| Field | Detail |
| --- | --- |
| Decision (Go / No-Go / Go with conditions) |  |
| Suspect patch package: integrity result |  |
| Backup & restore readiness |  |
| Rollback path confirmed |  |
| Outstanding blockers / conditions |  |
| Decided by / date |  |

- Finalize the backup, integrity, patch, WSUS and change-package records into the Module 2 evidence bundle.

## Deliverables - tick when complete and evidence is saved

☐  Maintenance Scope & Backup Scope

☐  Backup Checklists for all three systems

☐  Backup Register with full metadata; at least one restore tested

☐  Integrity Hash Record, including the suspect patch package result

☐  OT Patch Decision Framework and Patch Decision Log

☐  AI vs Vendor / OT Review

☐  WSUS Deployment & Maintenance Schedule

☐  Change Request and Rollback Plan

☐  Change Package Completeness Review

☐  Go / No-Go Decision with rationale

## Reflection Questions

The patch package hash did not match the claimed value. What does that allow you to conclude, and what does it not?

Which single missing piece of evidence would most change your go / no-go decision?

How does island-mode SIS isolation change the risk of an OT patch going wrong?

What is the minimum backup set that still lets you fully recover the engineering station?

## Evidence Log - Session 4

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |
| Session 4 sign-off |  |  |  |
| Trainee / Team | Date | Trainer sign-off | Evidence bundle ref |

# Module 2: Completion Record

To be signed once all four sessionly evidence bundles are complete and stored at <EVIDENCE_STORE>.

| Session | Deliverables complete (Y/N) | Evidence hashed & stored | Trainer initials | Date |
| --- | --- | --- | --- | --- |
| Session 1 |  |  |  |  |
| Session 2 |  |  |  |  |
| Session 3 |  |  |  |  |
| Session 4 |  |  |  |  |

## Sign-Off

| Field | Signature / notes |
| --- | --- |
| Trainee / team lead signature |  |
| Trainer signature |  |
| Overall Module 2 outcome / notes |  |