Module 3

Network Security for Operational Technology

Student Lab Workbook

Industrial Cybersecurity Training Program

| Trainee name |  |
| --- | --- |
| Team / Kit ID |  |
| Cohort / Class |  |
| Trainer |  |
| Start date |  |

# Contents

Contents	2

How to Use This Workbook	4

Working Method for Every Session	4

Evidence Naming Convention	4

Safety-First Rules (apply to the whole module)	5

Kit Parameter Sheet	6

Module 3 Learning Outcomes	7

Session 1: Network Behaviour and Anomaly Recognition............................................................ 7

Scenario	7

Learning Objectives	7

What you will use this session	7

Pre-Flight Checklist	8

Exercise Steps	8

Part A - Establish the Known-Good Packet Journey	8

Part B - Layer 2 and Layer 3 Anomaly Recognition	9

Part C - Transport and Application Behaviour	9

Part D - Unknown-PCAP Challenge and Control Requirements............................ 9

Deliverables - tick when complete and evidence is saved	10

Reflection Questions	10

Evidence Log - Session 1	11

Session 2: VLAN Segmentation, Switch Hardening and Industrial Firewall Enforcement	12

Scenario	12

Learning Objectives	12

What you will use this session	12

Pre-Flight Checklist	12

Exercise Steps	13

Part A - Risk-Linked Segment and Port Design	13

Part B - VLAN Implementation, Switch Hardening and Layer 2 Verification	13

Part C - Allowed Services and Deny-by-Default Firewall Enforcement	14

Part D - Control Validation, Local Evidence and Baseline Approval	15

Deliverables - tick when complete and evidence is saved	16

Reflection Questions	16

Evidence Log - Session 2	17

Session 3: Controlled Segmentation Security Testing and Remediation........................ 18

Scenario	18

Learning Objectives	18

What you will use this session	18

Pre-Flight Checklist	18

Exercise Steps	18

Part A - Baseline Freeze, Scope and Test Planning	18

Part B - Expected-Result Segmentation Testing	19

Part C - Configuration-Gap Localisation and Correction	20

Part D - Retest, Advanced Concept Observation and Residual Exposure	20

Deliverables - tick when complete and evidence is saved	21

Reflection Questions	21

Evidence Log - Session 3	22

Session 4: Secure IT/OT Convergence, Remote Access and DMZ Boundary Control	23

Scenario	23

Learning Objectives	23

What you will use this session	23

Pre-Flight Checklist	23

Exercise Steps	24

Part A - DMZ Services and Historian Flow Design	24

Part B - Remote Access and Boundary Flow Validation	24

Part C - Reverse-Path Validation and SIS Boundary Models.................... 25

Part D - Boundary Testing and Integrated Architecture	25

Deliverables - tick when complete and evidence is saved	26

Reflection Questions	26

Evidence Log - Session 4	27

Module 3: Completion Record	28

Sign-Off	28

# How to Use This Workbook

This workbook is your primary record of evidence for Module 3. You will complete it hands-on, in your team, using the Trainee Kit to analyse network behaviour, implement and validate VLAN and firewall segmentation, test approved communication paths, and design secure IT/OT boundary and remote-access controls. Trainer Rack demonstrations and Central infrastructure will support the activities. Every table you complete, configuration export, network diagram, packet capture, test result, screenshot, and hash you record becomes part of your assessed evidence bundle.

## Working Method for Every Session

- Read the scenario and objectives for the session before touching any tool.

- Complete the pre-flight checklist. Do not start the exercise until every item is ticked.

- Work through the numbered steps in order. Where a command, filter or prompt is given in a grey box, use it as written unless the trainer changes the placeholder value.

- Fill in the tables as you go, not from memory afterwards. Distinguish observed facts from assumptions.

- Capture evidence using the naming convention below and log it in the evidence table.

- Use Generative AI only where a step directs you to. Treat its output as material for your own analysis, never as a finding in itself: validate every claim against your own evidence - packet captures, device configurations, firewall and switch state, logs, advisories and the architecture. Record where the AI was wrong, unsupported or over-generalised.

- Get the trainer sign-off at the end of the session.

## Evidence Naming Convention

M3-W<session>-<KIT_ID>-<step>-<short-description>.<ext> Examples: M3-W1-KIT03-08-normal-vs-abnormal.pcapng M3-W2-KIT03-08-firewall-ruleset.txt M3-W3-KIT03-07-controlled-test-results.pcapng

Record every artefact in the Evidence Log at the end of each session. For any exported file, compute a SHA-256 hash and record it.

# Windows PowerShell: hash an evidence file Get-FileHash -Algorithm SHA256 '.\M3-W2-KIT03-06-firewall-baseline.cfg'

## Safety-First Rules (apply to the whole module)

- No changes without backup, rollback plan and trainer approval. In this module that specifically includes VLAN configuration, switch port assignments, firewall rule sets, jump-host settings and any remote-access configuration. Export and hash the running configuration before you change it.

- The SIS island is out of scope for any scan, write, injection, VLAN change or reachability test at all times. Validate its separation only through approved configuration, local device evidence and passive observation.

- Segment before you cut. When implementing deny-by-default, always confirm the legitimate flows first (from your matrices and captures) so that control traffic is not interrupted. A firewall rule that blocks cyclic PLC/I/O traffic is a safety event, not just a connectivity fault.

- Controlled security testing stays inside the signed scope. Student testing is limited to approved sources, targets, ports, rates and techniques. No credential attacks, exploit scripts, uncontrolled load, denial-of-service, persistence or testing outside the agreed scope.

- Stop and call the trainer if any device alarms, drops communication, changes state unexpectedly, or if you are unsure whether an action is safe.

- Generative AI is a challenger, not a source of truth. Never paste credentials, live IP schemes you were told to keep internal, firewall configurations, or client-identifying data into an external AI tool.

# Kit Parameter Sheet

Complete this sheet before starting the practical activities and update the student-designed fields when instructed. It provides the approved device addresses, network identifiers, infrastructure references, safety exclusions, and evidence locations used throughout the module.

Use these values whenever a workbook step, command, diagram, firewall rule, or evidence filename contains a placeholder. Confirm all trainer-assigned values before testing or making configuration changes.

| Placeholder | Value (fill in) | Description | Notes |
| --- | --- | --- | --- |
| <KIT_ID> |  | Your kit / team identifier | Used in evidence file names |
| <PLC_IP> |  | PCS 7 CPU 410 controller IP | Control zone |
| <ET200SP_IP> |  | ET 200SP I/O station IP | Control zone |
| <HMI_WS_IP> |  | Operator / engineering workstation IP | Control zone |
| <SC646_MGMT_IP> |  | SCALANCE SC646-2C firewall management IP | Zone boundary device |
| <XC208_MGMT_IP> |  | SCALANCE XC208G switch management IP | Access switch |
| <KIT_SUBNET> |  | Kit subnet / mask |  |
| <KIT_GW> |  | Kit default gateway |  |
| <OT_VLAN> |  | OT / control VLAN ID | Session 2 |
| <DMZ_VLAN> |  | OT DMZ VLAN ID | Sessions 2 and 4 |
| <SUPPORT_VLAN> |  | Maintenance / engineering VLAN ID | Session 2 |
| <ENT_VLAN> |  | Enterprise / IT VLAN ID | Reference |
| <JUMP_HOST_IP> |  | Hardened remote-access jump host IP | Session 4 |
| <SINEMA_RC_IP> |  | SINEMA Remote Connect server IP | Session 4 (Central) |
| <HISTORIAN_IP> |  | Process historian IP | Control / DMZ |
| <HIST_REPLICA_IP> |  | DMZ historian replica IP | Session 4 |
| <TR_SIS_SEG> |  | Trainer Rack SIS island segment / VLAN | OUT OF SCOPE for active work |
| <TR_PCAP_NAME> |  | Shared Trainer Rack PCAP file name | Distributed each session |
| <AI_TOOL> |  | Approved Generative AI tool / URL | Validation assistant only |
| <EVIDENCE_STORE> |  | Path / share for the evidence bundle |  |

# Module 3 Learning Outcomes

By the end of this module you will be able to:

- Classify OT assets into zones (Enterprise, DMZ, Control, SIS island) and justify each placement against IEC 62443 zone-and-conduit principles.

- Define conduits between zones with their protocols, ports, purpose and required boundary controls, and assign target security levels (SL-T) to each zone.

- Design and implement VLAN segmentation and deny-by-default industrial firewall rules on SCALANCE devices, and verify isolation with connectivity tests and packet evidence.

- Convert the approved communication policy into a formal segmentation test plan and execute controlled assurance tests from authorised Trainee Kit segments.

- Identify a trainer-introduced configuration weakness, apply the minimum safe correction after approval, and demonstrate the expected result through retesting.

- Design secure IT/OT convergence with DMZ services, historian replication, NGFW boundary control and protected SIS-island separation.

- Use Generative AI as a challenger for security-level and boundary-control reasoning and validate its output against architecture and packet evidence.

# Session 1: Network Behaviour and Anomaly Recognition

Total duration: 120 minutes

## Scenario

The previously established PCAP provides the known-good communication baseline. The trainer introduces representative Layer 2, Layer 3, Layer 4 and application-level anomalies one at a time. Your team must determine what changed, explain the underlying networking principle, identify the affected OSI layer, assess the OT consequence, and recommend an appropriate preventive or detective control. The objective is recognition and evidence-based interpretation, not full exploitation.

## Learning Objectives

- Establish a known-good baseline of IP addresses, MAC addresses, protocols, ports and communication pairs.

- Trace a normal OT flow across Ethernet, ARP, IP, TCP/UDP and the application protocol.

- Recognise representative Layer 2, Layer 3, Layer 4 and application-level anomalies from packet and timing evidence.

- Explain the networking principle exploited, the OT consequence, and suitable preventive or detective controls.

- Complete an unknown-PCAP recognition challenge containing one or two anomalies.

- Translate findings into preliminary requirements for segmentation, switch hardening, firewall restrictions, rate controls and monitoring.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Trainer Rack | Controlled, trainer-approved generation or demonstration of representative anomalies |
| Wireshark | Trace normal flows and compare packet, timing, session and payload evidence |
| SCALANCE XC208G / SPAN port | Provide the approved capture point and observe MAC-learning and broadcast behaviour |
| Normal and trainer-generated traffic captures | Compare known-good behaviour with selected hands-on, demonstrated or prepared anomalies |
| OSI-Layer Network Risk and Evidence Matrix | Map each condition to the affected layer, exploited principle, indicators, OT consequence and controls |
| Generative AI (<AI_TOOL>) | Challenge difficult interpretations only; validate every claim against packet and architecture evidence |

## Pre-Flight Checklist

☐ Approved capture point and SCALANCE XC208G SPAN port confirmed.

☐ Known-good PCAP and earlier-module inventory are available.

☐ Trainer-approved anomaly captures, trainer-operated traffic-generation scripts and load limits are available.

☐ OSI-Layer Network Risk and Evidence Matrix received.

☐ Rollback procedure, stop conditions and SIS exclusion are confirmed.

☐ Access to <AI_TOOL> confirmed.

☐ Evidence folder created at <EVIDENCE_STORE> using the naming convention.

## Exercise Steps

### Part A - Establish the Known-Good Packet Journey

- Review the known-good IP addresses, MAC addresses, protocols, ports and communication pairs from earlier modules.

- Trace one normal OT flow through Ethernet, ARP, IP, TCP or UDP, and the application protocol.

- Explain same-subnet communication, ARP resolution, switching, routing and default-gateway behaviour.

- Observe normal ARP requests and replies and switch MAC-learning behaviour. Record the packet and table evidence.

Table W1.1 - Annotated Normal-Flow Packet Journey

| Stage | Source → destination | Layer / protocol | Observed packet fields / behaviour | Role in the normal flow | Evidence reference |
| --- | --- | --- | --- | --- | --- |
| Ethernet |  | Layer 2 |  |  |  |
| ARP |  | Layer 2 |  |  |  |
| IP |  | Layer 3 |  |  |  |
| Transport |  | TCP / UDP |  |  |  |
| Application |  | OT protocol |  |  |  |

### Part B - Layer 2 and Layer 3 Anomaly Recognition

- The trainer introduces a controlled MAC-spoofing or ARP-poisoning condition. Identify duplicate or changing MAC/IP associations and explain the Layer 2 risk.

- Inspect a prepared IP-spoofing or abnormal ICMP sample. Compare source and destination addressing, ICMP types and codes, volume, direction and expected behaviour.

- Record each observation in the Normal-versus-Abnormal Evidence Worksheet and identify whether switch hardening, segmentation, rate controls or monitoring would reduce the risk.

Trainer-operated scenarios only. The trainer performs all active anomaly generation using approved scripts, bounded rates and defined stop conditions. Learners observe and analyse the resulting traffic. Prepared captures are used where live demonstration would create unnecessary operational risk.

### Part C - Transport and Application Behaviour

⚠ SAFETY All active conditions must use trainer-approved scripts, bounded rates and defined stop conditions. Do not target the SIS, do not generate uncontrolled load, and stop immediately if any device alarms, drops communication or changes state unexpectedly.

- Observe the trainer-generated bounded TCP connection condition and compare it with the known-good capture. Review connection attempts, TCP flags, completed and incomplete handshakes, resets, retransmissions and timing.

- Review the trainer-provided UDP-volume or timing anomaly. Compare packet rate, direction, duration, source and destination pairs, and the effect on expected OT communication.

- Review one trainer-generated or prepared application-level anomaly, such as an unexpected function type, write request, replayed message, new communicating node or unusual request frequency.

- Record the Layer 2, Layer 3, transport and application observations in Tables W1.2 and W1.3, including evidence, operational consequence and suitable controls.

Table W1.2 - Normal-versus-Abnormal Evidence Worksheet

| Condition / scenario | Known-good behaviour | Observed change / indicators | Affected layer / principle | OT consequence | Evidence reference |
| --- | --- | --- | --- | --- | --- |
| MAC / ARP condition |  |  |  |  |  |
| IP / ICMP condition |  |  |  |  |  |
| TCP condition |  |  |  |  |  |
| UDP condition |  |  |  |  |  |
| Application condition |  |  |  |  |  |

Table W1.3 - OSI-Layer Network Risk and Evidence Matrix

| Condition | OSI layer | Networking principle affected | Indicators / evidence | OT consequence | Preventive / detective controls |
| --- | --- | --- | --- | --- | --- |
| MAC / ARP manipulation | Layer 2 |  |  |  |  |
| IP / ICMP anomaly | Layer 3 |  |  |  |  |
| TCP connection anomaly | Layer 4 |  |  |  |  |
| UDP volume / timing anomaly | Layer 4 |  |  |  |  |
| Application request anomaly | Application |  |  |  |  |
| Unknown-PCAP finding |  |  |  |  |  |

### Part D - Unknown-PCAP Challenge and Control Requirements

- Complete the unknown-PCAP challenge and identify one or two anomalies without being told their type.

- For each finding, record the affected layer, packet or timing indicators, confidence, alternative explanation, recommended control and evidence reference in Table W1.4.

- Use <AI_TOOL> to challenge at least two interpretations. Validate each alternative against packet fields, timing and architecture evidence, and record the verdict in Table W1.5.

- Translate the validated findings into preliminary network-control requirements. Record the control family, requirement, planned validation evidence and safety constraint in Table W1.6.

Table W1.4 - Unknown-PCAP Findings

| Finding / suspected anomaly | Affected layer | Packet / timing indicators | Alternative explanation | Confidence (H/M/L) | Recommended control | Evidence reference |
| --- | --- | --- | --- | --- | --- | --- |

#### Suggested AI prompt

Challenge this packet interpretation using only the evidence provided. Identify the strongest alternative explanation, the specific packet fields or timing evidence that would distinguish the alternatives, and any assumptions that remain unsupported. Do not invent assets, flows or intent.

Table W1.5 - Interpretation Challenge Log

| Interpretation challenged | AI alternative / concern | Packet / architecture evidence checked | Verdict (kept / corrected / unresolved) | Reason |
| --- | --- | --- | --- | --- |

Table W1.6 - Preliminary Network-Control Requirements

| Validated finding / risk | Control family | Preliminary requirement | Architecture location | Planned validation method / evidence | Safety / scope constraint |
| --- | --- | --- | --- | --- | --- |

## Deliverables - tick when complete and evidence is saved

☐  Annotated normal-flow packet journey complete

☐  Normal-versus-abnormal evidence worksheet complete

☐  OSI-Layer Network Risk and Evidence Matrix complete

☐  Unknown-PCAP findings recorded with evidence and confidence

☐  Challenging interpretations validated against packet evidence

☐ Preliminary network-control requirements prepared

☐ Evidence Log updated; packet captures and screenshots named and hashed

## Reflection Questions

Which packet fields most clearly distinguished normal from abnormal behaviour?

Which observed condition presents the greatest OT consequence, and why?

Which control requirement is most directly supported by your packet evidence?

## Evidence Log - Session 1

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |

| Trainee / Team |  |
| --- | --- |
| Trainer sign-off |  |
| Date |  |
| Evidence bundle ref |  |

# Session 2: VLAN Segmentation, Switch Hardening and Industrial Firewall Enforcement

Total duration: 120 minutes

## Scenario

The established asset inventory, Purdue and operating-zone mapping, communication-flow evidence, configuration baseline and network-risk findings provide the design inputs for this activity. Your team must refine those inputs into an IEC 62443 zone-and-conduit design, assign justified target security levels, and implement the approved design using VLANs, switch hardening and deny-by-default industrial firewall rules. Legitimate OT communication must remain available while unnecessary paths and broadcast exposure are reduced. Every configuration and rule must trace to an approved flow or security requirement and be verified with measurable evidence.

## Learning Objectives

- Validate the imported asset, operating-zone, communication-flow, criticality and configuration evidence.

- Refine the existing groupings into defensible IEC 62443 security zones and justify any split, merge or reclassification.

- Assign and justify a target security level (SL-T) for each security zone.

- Define conduits with permitted services, direction, purpose and required boundary controls.

- Translate the approved zone-and-conduit design into VLANs, subnets, switch ports and management restrictions.

- Implement deny-by-default industrial firewall rules and validate the prioritised allowed and denied paths.

- Trace each requirement through configuration, rule, test, evidence and residual risk.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| SCALANCE XC208G | VLAN configuration, access and trunk port assignment, unused-port shutdown and supported MAC or port restrictions |
| SCALANCE SC646-2C | Deny-by-default firewall enforcement and local log review |
| Module 1 traffic-flow matrix and Session 1 risk matrix | Provide approved business flows and documented risks; refine rather than rebuild |
| Ping, ARP and switch tables | Validate same-VLAN and cross-VLAN reachability and Layer 2 isolation |
| Wireshark | Verify broadcast containment, rule outcomes and unexpected traffic |
| Approved PCAP and configuration backups | Confirm legitimate services, preserve rollback and compare pre/post-control evidence |

## Pre-Flight Checklist

☐  Kit powered up, XC208G and SC646-2C management reachable at <XC208_MGMT_IP> / <SC646_MGMT_IP>.

☐  Validated Module 1 traffic-flow matrix and Session 1 Network Risk and Evidence Matrix available.

☐  Current switch and firewall configurations exported and SHA-256 hashed BEFORE any change (rollback baseline).

☐  Approved addressing, access and trunk port roles, management access and allowed VLANs confirmed.

☐  Rollback plan agreed and trainer approval obtained for configuration changes.

☐  Local SC646-2C logging is enabled and accessible; SIS exclusion and safety stop conditions are confirmed.

## Exercise Steps

### Part A - IEC 62443 Zone, Conduit and Segmentation Design

- Validate the design inputs. Review the imported asset inventory, Purdue and operating-zone mapping, communication-flow matrix, criticality, threat-model and configuration evidence. Record any unresolved conflict before design work continues.

- Refine the security zones. Confirm whether each existing grouping is a defensible IEC 62443 security zone. Split, merge or reclassify only where security requirements, criticality, trust or access needs differ.

- Assign target security levels. Assign and justify SL-T for each approved zone using consequence, threat exposure and required resistance as the basis.

- Define the conduits. Convert validated communication flows into named conduits with source zone, destination zone, permitted services, direction, purpose and boundary control.

- Design the segmentation. Translate the approved zone-and-conduit model into VLAN IDs, non-overlapping subnets, gateways, access ports, trunks, allowed VLANs, management restrictions and unused-port state.

- Establish traceability. Map each zone or conduit requirement to the planned VLAN, switch control, firewall rule and validation test.

Table W2.1 – IEC 62443 Zone Refinement and SL-T Rationale

| Security zone | Imported grouping / assets | Refinement decision | Security rationale | SL-T | SL-T justification / evidence |
| --- | --- | --- | --- | --- | --- |

Table W2.2 – Conduit and Permitted-Services Register

| Conduit ID / name | Source zone | Destination zone | Protocol / port / direction | Operational purpose | Required boundary control | Evidence source / approval |
| --- | --- | --- | --- | --- | --- | --- |

Table W2.3 – Design Traceability Matrix

| Zone / conduit requirement | VLAN / subnet / port control | Firewall rule reference | Planned validation test | Expected result | Evidence reference |
| --- | --- | --- | --- | --- | --- |

### Part B - VLAN Implementation, Switch Hardening and Layer 2 Verification

- Verify the rollback baseline. Confirm the current XC208G and SC646-2C configuration exports, hashes, rollback method and trainer approval.

- Configure the VLAN design. Create the approved VLANs, access ports, trunk ports, allowed VLANs, PVIDs and restricted management access on the XC208G.

- Harden the switch ports. Disable unused ports, remove unnecessary trunking and apply supported MAC or port restrictions.

- Verify Layer 2 implementation. Confirm VLAN membership, PVIDs, switch state, same-VLAN behaviour, cross-VLAN isolation and broadcast containment.

- Record implementation evidence. Update the design traceability matrix with configuration and test references.

Reference - SCALANCE VLAN and isolation checks

# On XC208G (WBM / CLI): confirm VLAN membership and PVID show vlan (VLAN table: tagged / untagged members) show interface status (port PVID and link state) # From the workstation - allowed vs blocked paths ping <PLC_IP> # expected: reachable (same OT VLAN) ping <HISTORIAN_IP> # expected per design ping <ENT_host> # expected: BLOCKED across zones # Wireshark on the SPAN/access port - confirm containment vlan # inspect 802.1Q tags seen on the port eth.dst == ff:ff:ff:ff:ff:ff # broadcast scope per VLAN

Table W2.4 – Layer 2 Isolation Test Log

| Test ID | From → to (host / VLAN) | Test method | Expected result | Actual result | Pass / Fail | Evidence reference |
| --- | --- | --- | --- | --- | --- | --- |

### Part C - Allowed Services and Deny-by-Default Firewall Enforcement

- Confirm the permitted services. Review the conduit register and resolve ambiguous, unnecessary or unsupported flows before creating firewall rules.

- Draft the rule set. Create ordered rules with source zone, destination zone, service, direction, state, action, logging and exact conduit reference.

Table W2.5 – Draft Firewall Rule Set

| Order | Action | Source zone / object | Destination zone / object | Service / direction / state | Logging | Conduit / rationale |
| --- | --- | --- | --- | --- | --- | --- |

- Challenge the draft. Use <AI_TOOL> to identify over-broad rules, missing flows and unjustified services. Validate every suggestion against approved evidence.

Table W2.6 – AI Rule Review and Correction Log

| Draft rule / issue | AI suggestion / concern | Evidence checked | Risk introduced if accepted | Corrected rule / decision | Reviewer verdict |
| --- | --- | --- | --- | --- | --- |

- Obtain approval. Record trainer review, tighten or reject unsupported rules, and confirm the staged implementation and rollback sequence.

- Implement the allow rules. Apply approved specific rules first and verify device health after each stage.

- Enable deny-by-default last. Apply the final deny rule only after cyclic control flows are confirmed and roll back immediately if health checks fail.

### Part D - Control Validation, Local Evidence and Baseline Approval

- Execute the approved test set. Test prioritised allowed and denied paths and record the exact source, destination, method, expected result and actual result.

- Review local evidence. Correlate SC646-2C rule matches, Wireshark frames and switch state with each test result.

- Observe the trainer-operated condition. Compare one bounded pre-control and post-control network condition without targeting the SIS or generating learner-operated reconnaissance.

- Resolve failures. Where a result differs from policy, stop, identify the configuration or rule issue, obtain approval and repeat the test after correction.

Table W2.7 – Segmentation and Firewall Validation Results

| Test ID / requirement | Source → destination / service | Expected result | Actual result | Rule / config reference | Log / packet evidence | Pass / Fail / retest |
| --- | --- | --- | --- | --- | --- | --- |

- Record residual risk. Document risks not addressed by VLANs, switch hardening or firewall enforcement, including owner and required treatment.

- Approve the baseline. Record the final configuration exports, hashes, rollback reference and trainer approval.

Table W2.8 – Residual Risk Register

| Residual risk | Affected zone / conduit and evidence | Consequence | Treatment / compensating control | Owner | Status / follow-up |
| --- | --- | --- | --- | --- | --- |

Table W2.9 – Segmented-Network Baseline Approval

| Field | Recorded value / evidence |
| --- | --- |
| Approved zone/SL-T design reference |  |
| Conduit register reference |  |
| Final XC208G configuration file / hash |  |
| Final SC646-2C configuration file / hash |  |
| Rollback baseline / method |  |
| Tests passed / exceptions |  |
| Trainer approval / date |  |

## Deliverables - tick when complete and evidence is saved

☐ IEC 62443 Zone Refinement and SL-T Rationale completed

☐ Conduit and Permitted-Services Register approved

☐ Design Traceability Matrix completed from requirement through validation evidence

☐  Deny-by-default SC646-2C firewall rule base implemented and exported

☐  Before/after reachability matrix complete

☐  Local SC646-2C firewall-log evidence captured and linked to test results

☐ Segmented-network baseline, rollback evidence and residual-risk register approved

☐  Evidence Log updated; configs and captures named and hashed

## Reflection Questions

Which legitimate flow was easiest to miss when you wrote the deny-by-default rules, and how would missing it have shown up?

What did the firewall logs reveal that the ping tests alone did not?

Where was the AI's rule proposal too broad, and what in the PCAP let you tighten it?

## Evidence Log - Session 2

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |

| Trainee / Team |  |
| --- | --- |
| Trainer sign-off |  |
| Date |  |
| Evidence bundle ref |  |

# Session 3: Controlled Segmentation Security Testing and Remediation

Total duration: 120 minutes

## Scenario

An independent assurance exercise must verify that the approved zone, conduit, VLAN and firewall design enforces the intended communication policy. Working in student test and monitoring roles, your team will plan and perform bounded security tests from authorised Trainee Kit segments, collect packet and local-device evidence, identify one trainer-introduced configuration weakness, apply the minimum safe correction after approval, and demonstrate closure through retesting. The SIS island remains out of scope for all active testing and is assessed only through configuration and passive evidence.

## Learning Objectives

- Convert the approved zone, conduit and firewall policy into a controlled security test plan with defined test points, expected results, evidence and stop conditions.

- Perform approved same-zone and cross-zone connectivity, service and scoped discovery tests from authorised Trainee Kit segments.

- Classify each result as blocked and logged, blocked but not logged, allowed and logged, or allowed but not logged.

- Localise a trainer-introduced configuration weakness using switch, firewall, packet and local-log evidence.

- Propose and implement the minimum safe correction after trainer approval, then repeat the failed test and verify legitimate-flow regression.

- Document residual exposure and preserve or restore the approved final baseline with configuration hashes.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Isolated Trainee Kit | Authorised controlled security-test environment |
| SCALANCE XC208G | VLAN, access-port, trunk, PVID and switch-state evidence |
| SCALANCE SC646-2C | Firewall rule, decision and local-log evidence |
| Wireshark | Packet-level validation of permitted, denied and unexpected paths |
| Ping, ARP discovery and Nmap | Bounded connectivity, neighbour and approved TCP-connect testing within the signed scope |
| Approved zone/conduit design and rule baseline | Define expected exposure, prohibited paths and configuration references |
| Controlled Security Test Plan | Record authorised sources, targets, methods, rates, stop conditions and evidence |

## Pre-Flight Checklist

☐  Signed Rules of Engagement in place: scope, approved techniques, prohibited actions, stop conditions.

☐ Student test and monitoring roles assigned; evidence responsibilities agreed.

☐ Approved zone/conduit design, VLAN configuration and firewall rule baseline available and hashed.

☐ Approved test source, targets, ports, timing profile and capture points confirmed.

☐  SIS segment <TR_SIS_SEG> confirmed OUT OF SCOPE for any injected traffic.

☐ Pre-test switch and firewall configurations exported and hashed; rollback owner and device-health monitoring confirmed.

## Exercise Steps

### Part A - Baseline Freeze, Scope and Test Planning

⚠ SCOPE & SAFETY This session is bounded by the signed Rules of Engagement. Use only approved sources, targets, ports, timing profiles and TCP-connect or connectivity methods. No exploitation, credential testing, denial-of-service, uncontrolled load or testing of the SIS segment. Stop immediately for device alarms, communication loss or unexpected process-state change.

- Freeze the baseline. Export and hash the approved switch and firewall configurations and record the rollback method.

- Confirm the Rules of Engagement. Record approved sources, targets, ports, methods, rates, time limits, prohibited actions, stop conditions and SIS exclusion.

- Build the test plan. Convert the approved zone/conduit and firewall policy into defined allowed-path, denied-path, service and scoped-discovery tests.

- Define expected evidence. Record the expected result, rule/configuration reference, packet or local-log evidence, health check and trainer approval before execution.

Table W3.1 – Controlled Security Test Plan

| Test ID | Requirement / conduit | Authorised source / target | Method / protocol / port | Expected result | Evidence to capture | Rate / stop condition | Trainer approval |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Part B - Expected-Result Segmentation Testing

- Execute approved connectivity tests. Test same-zone communications, approved cross-zone conduits and prohibited cross-zone paths from the authorised source.

- Perform bounded discovery. Use only the approved ping, ARP-neighbour and slow TCP-connect parameters against listed targets and ports.

- Collect evidence. Correlate Wireshark frames, switch state and SC646-2C rule/log evidence with each test.

- Classify the outcome. Record blocked and logged, blocked but not logged, allowed and logged, allowed but not logged, or inconclusive; stop and escalate any unexpected result.

Reference - scoped reconnaissance (approved options only)

# Host discovery only - no exploitation, no aggressive scripts ping <KIT_SUBNET_broadcast> # quick liveness sudo arp-scan --interface=eth0 --localnet # L2 neighbour discovery nmap -sn <KIT_SUBNET> # ping sweep, no port scan nmap -sT -p <approved-ports> <target> # TCP connect on approved ports only # PROHIBITED in this session: -sS floods, -T5 against control assets, # NSE exploit/dos scripts, any scan of <TR_SIS_SEG>.

Table W3.2 – Controlled Security Test Results

| Test ID | Test performed | Expected result | Actual result | Outcome classification | Switch / firewall evidence | Packet evidence | Pass / Fail / Inconclusive |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Part C - Configuration-Gap Localisation and Correction

- Identify the introduced weakness. Use the failed or unexpected test to locate the incorrect access-port VLAN, excessive trunk allowance, overly broad or shadowed rule, or unintended management exposure.

- Confirm the root cause. Correlate configuration, packet and local-log evidence and record the failed test reference.

- Plan the minimum safe correction. Record the exact change, expected effect, pre-change configuration hash, rollback trigger and rollback action.

- Challenge the diagnosis if needed. Use <AI_TOOL> only for difficult interpretations and validate the suggestion against evidence.

- Implement after approval. Apply the approved correction, verify device health, export and hash the post-change configuration.

Table W3.3 – Configuration Gap, Correction and Approval Record

Any 'allowed/invisible' result is the highest-priority finding: the control failed and you would not have known. Mark these clearly for Part D.

### Part D - Retest, Advanced Concept Observation and Residual Exposure

- Repeat the failed test and confirm the corrected result with packet and local device evidence.

- Observe one trainer-demonstrated advanced bypass concept in an isolated or prepared environment; do not reproduce it.

- Document residual design limitations and controls requiring later monitoring.

- Restore or preserve the approved Session 3 baseline.

Table W3.4 - Gap Log (no-AI)

| Gap | How it got through / was missed | Which control should have caught it | Priority |
| --- | --- | --- | --- |

Table W3.5 - Rule Refinement and Retest

| Rule changed | Change made | Gap it closes | Red retest result | Confirmed closed? (Y/N) |
| --- | --- | --- | --- | --- |

Table W3.6 - Role-Swap Creative Bypass

| Bypass attempted |  |
| --- | --- |
| Outcome (which quadrant) |  |
| New gap identified? |  |
| Recommended follow-up |  |

## Deliverables - tick when complete and evidence is saved

☐  Rules of engagement

☐  Segmentation test plan and expected-versus-actual results matrix

☐  Local evidence pack and configuration-gap record

☐  Corrective-change and retest record

☐  Residual exposure summary

☐  Role-swap creative bypass recorded with outcome

☐  Evidence Log updated; captures, logs and configs named and hashed

## Reflection Questions

Which Red action fell into 'allowed/invisible', and what would have made it visible?

Did the SIS island resist every attempt to reach it? What in your evidence shows that?

Which of the three refined rules gave the biggest reduction in successful bypasses, and why?

## Evidence Log - Session 3

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |

| Trainee / Team |  |
| --- | --- |
| Trainer sign-off |  |
| Date |  |
| Evidence bundle ref |  |

# Session 4: Secure IT/OT Convergence, Remote Access and DMZ Boundary Control

Total duration: 120 minutes

## Scenario

A vendor requires temporary remote access to an OT engineering service, while historian data must be exposed to enterprise users and selected monitoring data may leave a protected environment through a one-way path. Students must design, trace, and validate the permitted paths without creating direct enterprise-to-control or reverse SIS access.

## Learning Objectives

- Catalogue required DMZ services and trace historian-to-replica and replica-to-enterprise flows.

- Define the remote-access chain through authentication/MFA, SINEMA Remote Connect, a jump host or controlled endpoint, and the authorised target.

- Validate approval, least privilege, session timeout, termination, emergency revocation, and local audit records.

- Build and test a Boundary Flow Matrix for remote access, historian exchange, management, and monitoring egress.

- Verify permitted one-way transfer and demonstrate that return traffic or a reverse session cannot be established.

- Distinguish a fully isolated SIS from an SIS architecture with approved one-way monitoring egress.

## What you will use this session

| Tool / Platform | Purpose |
| --- | --- |
| Trainer Rack architecture | Integrated enterprise–DMZ–OT–SIS design and control-point tracing |
| DMZ services and historian replica | Controlled enterprise exposure without direct enterprise-to-OT access |
| SINEMA Remote Connect | Approved temporary remote-session establishment, termination, and local audit validation |
| Jump host / controlled endpoint | Least-privilege path to the authorised OT target |
| Data diode / unidirectional gateway demo | Permitted outbound transfer and reverse-path validation |
| Boundary Flow Matrix | Document initiator, destination, protocol, port, direction, purpose, controls, and tests |

## Pre-Flight Checklist

☐  Session 1 zone model and Session 2/3 firewall state to hand.

☐  Trainer Rack architecture with DMZ, historian and SINEMA RC reference available.

☐  Central infrastructure access confirmed: SINEMA RC <SINEMA_RC_IP>, Splunk <SPLUNK_IP>, data-diode demo <DIODE_ID>.

☐  Historian <HISTORIAN_IP> and DMZ replica <HIST_REPLICA_IP> identified.

☐  Access to <AI_TOOL> confirmed.

## Exercise Steps

### Part A - DMZ Services and Historian Flow Design

- Review the Session 3 validated OT segmentation baseline and identify services that genuinely require exchange between enterprise, DMZ, and OT.

- Classify each service by initiator, destination, protocol, port, direction, sensitivity, and operational purpose.

- Design DMZ placement for the historian replica, remote-access service, and approved intermediaries.

- Trace historian-to-replica and replica-to-enterprise flows and confirm that enterprise systems do not connect directly to OT sources.

Table W4.1 - DMZ Service Inventory

| DMZ service | Function | Data sensitivity | Talks to (zones) | Criticality | Connection initiated from |
| --- | --- | --- | --- | --- | --- |

### Part B - Remote Access and Boundary Flow Validation

- Define the remote-access chain: external user, authentication/MFA, SINEMA Remote Connect, jump host or controlled endpoint, and target system.

- Define approval, duration, least privilege, session timeout, termination, and emergency revocation requirements.

- Establish one trainer-approved session and verify the actual path, permitted target, session termination, and local SINEMA RC/session audit records.

- Attempt one unauthorised target or expired-access condition and confirm that it is denied.

- Build the Boundary Flow Matrix for remote access, historian exchange, management, and monitoring egress.

Table W4.2 - Boundary Flow Matrix

| Flow (from → to) | Trust boundary crossed | Carries | Direction | Control enforcing it | How the control is tested |
| --- | --- | --- | --- | --- | --- |

### Part C - One-Way Transfer, Reverse-Path Validation and SIS Models

- Observe the data-diode/unidirectional-gateway demonstration and verify that permitted outbound data crosses the boundary.

- Verify that return traffic or a reverse session cannot be established.

- Distinguish a fully isolated SIS from an SIS architecture with approved one-way monitoring egress; do not claim that a diode automatically protects an islanded SIS.

- Map required firewall, intermediary, authentication, and one-way controls to each boundary flow.

⚠ SAFETY A unidirectional boundary protects only when there is genuinely no return path. In the design, confirm that there is no management, acknowledgement, or temporary bidirectional channel around the diode—these are the reverse paths that must be excluded.

Table W4.3 - Egress / Diode Trace and SIS Protection

| Where the SINEMA RC session terminates |  |
| --- | --- |
| Location of the one-way boundary |  |
| What is permitted to egress (and format) |  |
| What is blocked inbound by the diode |  |
| How SIS island mode + diode combine |  |
| Any residual/return path risk |  |

### Part D - Boundary Testing and Integrated Architecture

- Produce and execute a small boundary-rule test procedure covering the approved flows and at least one denied condition.

- Create one integrated enterprise–DMZ–OT–SIS architecture showing permitted directions, remote-access controls, intermediaries, and one-way control points.

- Document setup and reset requirements for SINEMA RC accounts, temporary approvals, DMZ services, diode test data, local audit access, rollback, and SIS safeguards.

#### Suggested AI prompt

I will provide a list of remote-access sessions from a SINEMA Remote Connect / Splunk export: source, user, start time, duration, destination zone, and frequency. Act as a reviewer of remote-access session patterns. Identify: – Sessions or patterns that are anomalous relative to the rest (unusual time, duration, source, or frequency). – Any session that appears to reach a destination zone it should not, based on the boundary model I provide. – For each observation: the specific session field(s) that support it, and what I should verify in the raw Splunk events. Do not assume sessions, users or destinations I have not provided, and do not label a pattern malicious without stating the evidence. Where the data is insufficient, say what additional log field is needed. I will verify each observation against the raw events.

Table W4.4 - SINEMA RC Log Validation and AI Session Review

| AI observation | Session field(s) cited | Confirmed in Splunk? (Y/N) | Verdict (real / benign / AI error) |
| --- | --- | --- | --- |

Table W4.5 - Boundary-Rule Test Procedure

| Boundary flow | Test performed | Expected result | Reverse-path test (must fail) | Pass / fail |
| --- | --- | --- | --- | --- |

Table W4.6 - Secure Convergence Summary

| DMZ service model |  |
| --- | --- |
| Boundary controls (by trust boundary) |  |
| Unidirectional egress design |  |
| SIS protection architecture |  |
| Open risks / next steps |  |

## Deliverables - tick when complete and evidence is saved

☐  DMZ service catalogue and remote-access control profile

☐  Boundary Flow Matrix and remote-session validation record

☐  One-way transfer and reverse-path evidence

☐  Boundary test procedure and integrated secure-convergence architecture

☐  Boundary-rule test procedure including reverse-path tests

☐  Secure Convergence Summary complete

☐  Evidence Log updated; diagrams, exports and screenshots named and hashed

## Reflection Questions

Which DMZ service creates the most trust-boundary exposure, and how is it controlled?

What in your design guarantees there is no reverse path around the unidirectional boundary?

What did the AI session analysis add over reading the Splunk events directly, and where did it overreach?

## Evidence Log - Session 4

| Evidence file name | Step | What it shows | SHA-256 (first 12 chars) |
| --- | --- | --- | --- |

| Trainee / Team |  |
| --- | --- |
| Trainer sign-off |  |
| Date |  |
| Evidence bundle ref |  |

# Module 3: Completion Record

To be signed once all four weekly evidence bundles are complete and stored at <EVIDENCE_STORE>.

| Session | Deliverables complete (Y/N) | Evidence hashed & stored | Trainer initials | Date |
| --- | --- | --- | --- | --- |
| Session 1 |  |  |  |  |
| Session 2 |  |  |  |  |
| Session 3 |  |  |  |  |
| Session 4 |  |  |  |  |

## Sign-Off

| Trainee / team lead signature |  |
| --- | --- |
| Trainer signature |  |
| Overall Module 3 outcome / notes |  |